.po 10

.he#Turbo DOS 1.42 8086 Update                T-Function 45#

.fo#Programmer's Guide         5-46-2.#       MAKE FILE#

Turbo DOS 1.42 Update

Prepared by Advanced Digital Corporation

________________________________________________________________

#T-Function 45     Make File#

Entry Arguments     |-------------------------------------------|

| Reg |        Description                  |

|-------------------------------------------|

|  CL   = 45                                |

|DS:DX  = FCB Address                       |

|  CH   = sharing vector (see explanation)  |

|___________________________________________|

Returned Value      |-------------------------------------------|

| Reg |        Description                  |

|-------------------------------------------|

| AL  | =  0 if successful                  |

| AL  | = -1 if directory full, file exists,|

|     |      or FCB invalid.                |

|___________________________________________|

Explanation         The Make File function creates a new (empty)

file specified by the FCB drive, name, type,

and extent fields (bytes 0 through 12).  Nor-

mally, the extent field (byte 12) should be

set to zero.  The directory entry for the new

file is placed under the current user number.

All file attributes are initialized to zero.

A request to make a file that already exists

is denied.

The newly created file is left in an open

state.  FCB interface bits F5 and F6 are

ignored.  The file is left open in either the

exclusive or permissive mode, depending on

the state of the compatibility flag bit 7.

Fil� sharing is determined by the sharing

vector passed in the CH register per the

following:

|----|----|----|----|----|----|----|----|

| 7  | 6  | 5  | 4  | 3  | 2  | 1  | 0  |

| DR | DW | DC | DP | AR | AW | AC | AP |

|---------------------------------------|

|     SHARING       |    ACCESS         |

|___________________|___________________|

Sharing bits:

Bit 7     DR   Deny Read Privilege�                    Bit 6     DW   Deny Write Privilege

Bit 5     DC   Deny Compatibility Privilege

Bit 4     DP   Deny Permissive Privilege

Bit 3     AR   Access Read Granted

Bit 2     AW   Access Write Granted

Bit 1     AC   Access Compatibility Granted

Bit 0     AP   Access Premissive Granted

The calling program should zero the FCB cur-

rent record field (byte 32) before doing

sequential reads or writes on the file.

Note: this new T-function provides superset

compatibility with MSDOS class software.  It

does not effect or otherwise preclude current

software utilizing C-function 22 (Open File).

###############################