NC-PROBE                                                           08/96

------------------------------------------------------------------------

Copyright by:                                                   RzK GmbH

PO Box 1200

All rights                                          D-53563 Asbach Ww

reserved!                                                    Germany

reserved!                                       Phone 49 2683 940000

FAX   49 2683 4537

http://www.rzk.com

Mail: info@rzk.com

------------------------------------------------------------------------

NC-Probe is free software.

NC-PROBE transmits network statistics in UDP frames. This PC-DOS software

needs a network card with packet driver interface and the statisticdriver

STATIPKT from RzK.

Before executing NC-PROBE the packet driver and STATIPKT has to be loaded:

Usually         <packetdriver> <packet_int> <int_level> <io_addr>

and then        STATIPKT [-i] <packet_int> <new_packet_int>

The parameter -i tells STATIPKT to generate statistics for IP-addresses

instead of MAC-addresses.

If you are using WfW 3.11 or Windows95 you can use the Windows VxD NDIS3PKT

instead of a packet driver for your network card. NDIS3PKT allows packet

driver applications to be used without loading any real-mode software

components. To download at ftp:/hsdndev.harvard.edu/ndis3pkt.

NC-Probe usage:

---------------

NC-PROBE [configfile]

If no file is specified NC-PROBE.CFG is used.

Example configfile:

-----------------------------

; NC-PROBE configurationfile:

; ---------------------------

194.127.156.156 ; Source IP address.

255.255.255.224 ; IP Netmask.

194.127.156.129 ; Router IP address.

194.127.156.150 ; Destination IP address.

0100FB585858    ; Destination MAC address, overwritten by ARP.

50              ; Source IP Port.

50              ; Destination IP Port.

1               ; Probenumber (1..300).

10              ; Intervallength in seconds.

At first NC-Probe sends an ARP request to determine the destination

MAC-address. If the destination IP-address is not in the own IP-net the

ARP is sent to the router address. If there is no reply the destination

MAC address given in the configfile is used.

Structure of statistic frames sent:

-----------------------------------

The following data is sent directly behind the UDP header:

Start Length Content

----- ------ ------

1     2    mark GH  (x4748)

3     1    mark 2   (x7D)

4     1    statistic-status-byte

5     2    probe mark  "|P"  (x7C50)

7     1    MAC/IP tag:  MAC-address statistic : "M" (x4D)

or for IP-addresses statistic: "P" (x50)

8     3    probe number  " 01" to "300"

11     1    unused

12     1    frame number

13     4    # received packets

14     4    unused

18     4    unused

22     4    netload in ms

26     4    unused

30     2    # packets < 64 Bytes

32     2           64 - 127 Bytes

34     2          128 - 255 Bytes

36     2          256 - 511 Bytes

38     2          512 - 1023 Bytes

40     2              > 1023 Bytes

42     1    # address entries in the following communication matrix = n

43  nx16    entries, each 16 Bytes:

<address_1> <address_2> <#packets_2_->_1> <#packets_1_->_2>

6 Bytes     6 Bytes     2 Bytes          2 Bytes

Depending on the STATIPKT call address entries refer to MAC addresses

or -STATIPKT with parameter "-i"- to IP-addresses (byte 1-4) and

IP-services (byte 5-6).

Counters for received frames, netload and lengthstatistics are

incremented permanently modulo 2^32 or 2^16.

A statistic frame is sent:

- at the end of each interval  or

- if the packet can't hold more statistical address entries (max. 88).

Recommenable is the usage of Net-Control for viewing, recording and

monitoring the statistical data sent by NC-Probe. Net-Control is

available in different versions for all sizes of networks. Please

mail RzK to receive detailed information about Net-Control.