HiMOVE (tm)

_________________________________________________________________

Upper

Memory

Manager

Version 2.14

C.SITTE SOFTWARETECHNIK

Postfach 42

A-5025 Salzburg/Austria

REMARKS

The HiMOVE (tm) Memory Management Software could be installed  on

computers  with  286  NEAT CHIPSet,  NEATsx  CHIPSet  and  AT/386

CHIPSet. It is also possible to use this software with any  other

type  of  upper  memory hardware. This manual  is  based  on  the

software revision V2.14 of November 1990.

SHAREWARE INFORMATION

HiMOVE (tm) is distributed under the shareware marketing concept.

This  means that you can try the program for a limited amount  of

time  to see if it meets your needs. After the trial period,  you

are required to register and pay the registration fee.

LICENSE

Non-registered users are granted a limited license to try HiMOVE

for  a  period  of four weeks. After the trial  period  your  are

required  to  register and to pay the registration fee.  You  are

encouraged to freely copy and distribute HiMOVE provided that:

1. No fee beyond normal media, duplication, and shipping costs

may be charged.

2. It must be copied in unaltered form complete with this license

information

Commercial  shareware  distributors  may  distribute  HiMOVE (tm)

subject  to  the above conditions only  after  obtaining  WRITTEN

permission from C.Sitte Softwaretechnik.

WARRANTY

C.Sitte Softwaretechnik makes no warranty of any kind, express or

implied,   including  without  limitation,  any   warranties   of

merchantability and/or fitness for a particular purpose.  C.Sitte

Softwaretechnik  shall  not be liable for  any  damages,  whether

direct, indirect, special or consequential arising from a failure

of  this  program to operate in the manner desired by  the  user.

C.Sitte  Softwaretechnik  shall not be liable for any  damage  to

data  or property which may be caused directly or  indirectly  by

use of the program.

In no event will C.Sitte Softwaretechnik be liable to you for any

damages,  including  any  lost profits,  lost  savings  or  other

incidental or consequential damages arising out of the use of  or

inability  to use these program, even if C.Sitte  Softwaretechnik

has  been advised of the possibility of such damages, or for  any

claim by any other party.

TRADEMARKS

IBM and AT are registered trademarks of IBM Corporation.

NEAT, CHIPSet, NEATsx are trademarks of Chips & Technologies Inc.

HiMOVE is a trademark of C.Sitte Softwaretechnik

EDITION

November 1990

Copyright 1989,1990 by C.Sitte Softwaretechnik

All rights reserved

NOTICE

The  information  in this document is subject to  change  without

notice. C.Sitte Softwaretechnik assumes no responsibility for any

errors that may appear in supplied materials or programs.

FUTURE RELEASES

The  next  majore release is expected at June/91.  The  following

modifications are planned:

- Improved program loading (more efficient usage of memory gaps)

- Additional optimizer phase for automatic program list

generation

- Move memory areas for BUFFERS statement into upper memory

- Bug fixes as necessary

ORDER INFORMATION

The  registration  fee for a single copy of HiMOVE  is  ATS 450.-

(exchange  rate  US$  to  Austrian Schillings  is  about  $1.-  =

ATS 11.- at November 90). Payment of this fee entitles you to:

*  A disk with the latest version of the HiMOVE memory manager,

which does not display the evaluation notice screen.

*  A printed and bound manual

*  Free support by electronic mail or by mail for the first six

month after payment.

*  Additional disk labels for the number of ordered copy licences.

*  Notification of the next major upgrade.

The  following per-unit prices apply for registration of  one  or

more copies of HiMOVE on the same order form:

1   ATS 450.-  (approx. US$ 40.- at Nov/90)

2 -  4   ATS 405.-  (-10%)

5 -  9   ATS 382.-  (-15%)

10 - 24   ATS 360.-  (-20%)

25 - 49   ATS 337.-  (-25%)

50 - 99   ATS 315.-  (-30%)

100+      (negotiable)

It  is also possible to obtain an additional site  licence  (copy

license).  A  minimum  of one disk  with  documentation  must  be

ordered. The licensee is responsible for duplicating the diskette

and documentation as needed, and for ensuring that the number  of

installed copies is not exceeded. Additional disk labels for  the

number of ordered copy licences will be delivered.

1 - 9    ATS 292.-  (-35%)

10 - 24   ATS 270.-  (-40%)

25 - 49   ATS 247.-  (-45%)

50 - 99   ATS 225.-  (-50%)

100 - 249  ATS 202.-  (-55%)

250 - 499  ATS 180.-  (-60%)

500+       (negotiable)

An additional shipping charge of ATS 80.- (ATS 40.- for  Austrian

orders) must be added to the total amount.

Payments  could  be  made by credit  card  (MC/Visa/AmEx/JCB)  or

eurocheque.

The accompanying file ORDER.FRM includes a detailed order form.

Every registered user receives a special offer for the next major

upgrade when it is available and information about new products.

T A B L E   O F   C O N T E N T S

1.    GENERAL. . . . . . . . . . . . . . . . . . . . . . . .    1

1.1     FIRST MEGABYTE . . . . . . . . . . . . . . . . . . .    1

1.2     NEW ENHANCED AT CHIP SET (NEAT)  . . . . . . . . . .    1

1.3     DEFINITION OF TERMS. . . . . . . . . . . . . . . . .    1

1.4     HiMOVE CONCEPT . . . . . . . . . . . . . . . . . . .    2

2.    INSTALLATION . . . . . . . . . . . . . . . . . . . . .    2

2.1     Shareware Copy from BBS or Disk Vendor . . . . . . .    2

2.1.1     Installation Procedure . . . . . . . . . . . . . .    3

3.    CONFIGURATIONPROGRAM "HISETUP.EXE" . . . . . . . . . .    4

3.1     Commandline Arguments. . . . . . . . . . . . . . . .    4

3.1.1     Examples . . . . . . . . . . . . . . . . . . . . .    5

3.2     Configuration File . . . . . . . . . . . . . . . . .    5

3.3     MAIN MENU  . . . . . . . . . . . . . . . . . . . . .    5

3.4     MENUFUNCTION "MEMORY AREAS". . . . . . . . . . . . .    6

3.4.1     Allocation Texts . . . . . . . . . . . . . . . . .    8

3.4.2     Printout of memory areas . . . . . . . . . . . . .    9

3.5     MENUFUNCTION "HELP". . . . . . . . . . . . . . . . .   10

3.6     MENUFUNCTION "OPTIONS" . . . . . . . . . . . . . . .   12

3.6.1     OPTION 1 - "Configuration Mode". . . . . . . . . .   12

3.6.2     OPTION 2 - "Prompt on Installation". . . . . . . .   13

3.6.3     OPTION 3 - "Installation Messages" . . . . . . . .   13

3.6.4     OPTION 4 - "Open Upper Memory" . . . . . . . . . .   13

3.7     MENUFUNCTION "PROGRAMLIST" . . . . . . . . . . . . .   14

3.8     MENUFUNCTION "RESIDENT PROGRAMS" . . . . . . . . . .   15

3.8.1     Description  . . . . . . . . . . . . . . . . . . .   15

3.8.2     Print Function . . . . . . . . . . . . . . . . . .   17

3.9     MENUFUNCTION "DOS MEMORY BLOCKS" . . . . . . . . . .   18

3.9.1     Description of header line . . . . . . . . . . . .   18

3.10    MENUFUNCTION "INSTALLATION OF MEMORY MANAGER". . . .   20

3.11    MENUFUNCTION "EXIT". . . . . . . . . . . . . . . . .   21

4.    MEMORY MANAGER "HIMOVE.SYS". . . . . . . . . . . . . .   22

4.1     Upload of resident programs. . . . . . . . . . . . .   23

4.2     Upload of device drivers via "HILOAD.SYS". . . . . .   24

5.    NEAT CHIPSet . . . . . . . . . . . . . . . . . . . . .   25

5.1     286 NEAT CHIPSet . . . . . . . . . . . . . . . . . .   25

5.1.1     Configuration of RAM Relocation (NEAT Setup) . . .   25

5.2     AT/386 CHIPSet . . . . . . . . . . . . . . . . . . .   25

5.3     CS8281 NEATsx CHIPSet. . . . . . . . . . . . . . . .   25

6.    QUICK INSTALLATION HELP. . . . . . . . . . . . . . . .   26

7.    To help you - help us! . . . . . . . . . . . . . . . .   27

8.    SUBJECT INDEX. . . . . . . . . . . . . . . . . . . . .   28

L I S T   O F   F I G U R E S

-----------------------------

3-1   Main Menu. . . . . . . . . . . . . . . . . . . . . . .    5

3-2   Memory Areas 512kB-768kB . . . . . . . . . . . . . . .    6

3-3   Memory Areas 768kB-1MB . . . . . . . . . . . . . . . .    7

3-4   Help Screen  . . . . . . . . . . . . . . . . . . . . .   10

3-5   Help Screen of Helpsystem  . . . . . . . . . . . . . .   11

3-6   Installation Options . . . . . . . . . . . . . . . . .   12

3-7   Programlist. . . . . . . . . . . . . . . . . . . . . .   14

3-8   Resident Programs (without HiMOVE) . . . . . . . . . .   15

3-9   Resident Programs (HiMOVE installed) . . . . . . . . .   16

3-10  Memory Blocks (without HiMOVE) . . . . . . . . . . . .   18

3-11  Memory Blocks (HiMOVE installed) . . . . . . . . . . .   19

3-12  End of Installation  . . . . . . . . . . . . . . . . .   20

HiMOVE (tm) Upper Memory Manager                            V2.14

1.  GENERAL

In 1981, the 1-megabyte address space of the newly introduced IBM

PC  seemed spacious, indeed. Less than a decade  later,  however,

that same amount of space seems claustrophobic. DOS users  search

continuously  for any nook or cranny into which they can stuff  a

few kilobytes of valuable RAM, to provide a little more breathing

room   for  today's  insatiable  application  programs,   network

drivers, and TSR programs.

1.1  FIRST MEGABYTE

The  first megabyte of DOS memory can be viewed as consisting  of

16 segments of 64 kBytes (kilo Bytes). The first 10 segments  are

reserved for the RAM into which you load DOS, device drivers, and

application  programs.  These  10 segments -  from  address  0000

through  address 9FFF - make up conventional memory  (640 kByte).

The  next  two segments (A000 and B000) are  reserved  for  video

buffers.  Hard  disk drive and video controller  ROMs  are  often

located  in the C000 segment. The 128 kByte of space in the  D000

and E000 segment are usually available for expanded-memory cards,

network-interface-cards, and the like. And last, the ROM BIOS  is

located in the F000 segment. Not all segments are used in a given

machine.

1.2  NEW ENHANCED AT CHIP SET (NEAT)

In 1988, Chips & Technologies introduced the NEAT chip set, a set

of  four VLSI ICs that, among other things, have built-in  shadow

RAM support. NEAT system boards can map memory from the  extended

region  to the upper region. The primary purpose of this type  of

mapping is to enable ROM shadowing (in which the contents of slow

ROMs  are copied to faster RAMs and executed from there).

1.3  DEFINITION OF TERMS

'Conventional  memory' is the first 640 kByte (655,360  bytes  of

RAM). It is not possible to use all of it for applications, since

DOS and various resident programs consume some of it. The  amount

of  available  memory  is  important,  because  PC  software  can

normally run only when it's in conventional memory.

Meanwhile, the 384 kByte between 640kB and 1MB (1024kB) is called

'upper  memory'.  RAM above 1 MByte is called  'extended  memory'

(not  'expanded memory', which swaps pages of RAM in and  out  of

conventional  memory  or  upper memory). The  first  (or  lowest)

64 kByte  of extended memory is called 'high memory'  and  should

not be confused with upper memory.

1

HiMOVE (tm) Upper Memory Manager                            V2.14

1.4  HiMOVE CONCEPT

HiMOVE  uses the mapping capability of the NEAT chip set to  load

device drivers and resident programs (TSR's) into unused areas of

the  upper memory segments. This increases the size of  the  free

conventional memory area. The following programs could be  loaded

into the upper memory area:

o DOS device drivers

o Command interpreter shell (COMMAND.COM)

o Memory resident programs (network shell, TSR's)

The HiMOVE memory manager could be installed and configured in an

easy  menu  driven way. No manual editing of the  CONFIG.SYS  and

AUTOEXEC.BAT files is necessary.

2.  INSTALLATION

2.1  Shareware Copy from BBS or Disk Vendor

If  you've  downloaded  an unregistered copy  of  HiMOVE  from  a

bulletin board (BBS) or an on-line service, or obtained a copy on

diskette  from  a disk vendor, you may try it for four  weeks  as

specified  in  the license agreement. After four weeks  you  must

either  register  HiMOVE  or  stop  using  it.  This  method   of

marketing, known as "Shareware", permits you to try HiMOVE before

you  pay  for  it. The four week evaluation  period  is  a  legal

requirement  based  on  the license agreement, NOT  a  time  bomb

inserted  in the program! HiMOVE will not stop working  or  cause

any  damage to itself or your system at the end of the four  week

period.

Shareware copies of HiMOVE may be "packaged" in a variety of ways

depending  on who distributed them. Regardless of packaging,  the

following files should be included:

HIMOVE.BAT      Batchfile for start of configuration

HIMOVE.SYS      HiMOVE Memory Manager

HILOAD.SYS      Device Driver Loader

HISETUP.EXE     Configuration Program

HIHELP.DAT      Helpfile (Part of HISETUP)

HIHELP.HLP      -"-

HISETUP.DEF     Default Configuration File

HISETUP.CFG     Configuration File

HIMOVE.DOC      Program Documentation

OVERVIEW.DOC    Short Program Overview

ORDER.FRM       Order Form

READ.ME         Last Minute Information

2

HiMOVE (tm) Upper Memory Manager                            V2.14

2.1.1  Installation Procedure

o      Create  directory  "\HIMOVE" or use any  other  convenient

directory name.

o      Copy distribution file(s) into directory "\HIMOVE". If the

files  are "packaged" into a compressed archive file,  use

the appropriate unpacking method to extract the files.

Examples:

HIMOVE.ZIP   ->  PKUNZIP HIMOVE

HIMOVE.LZH   ->  LHARC -E HIMOVE

HIMOVE.EXE   ->  HIMOVE  (Self Extracting archive)

o      Enter "HIMOVE" to start the configuration batchfile.

o      Use the menu function "Memory areas  F2" to get a  display

of  the actual memory allocation of your computer.  Verify

the suggested assignments of <RAM1...RAMxx> to the  unused

memory areas and change them if necessary.

o      Use the menu function "Installation of Memory Manager  F9"

to install the software on your computer.

o      Context  sensitive help information is available  most  of

the time by pressing the "F1" key.

o      At the end of the configuration program, you must  restart

the computer (press CTRL-ALT-DEL or use RESET button).

o      After  restart, the memory management software is  in  the

"Configuration Mode",  where  you will be asked  at  every

start of a program, if this program should be loaded  into

the upper memory area or not.

o      At  the end of the configuration phase, the  configuration

batchfile  "HIMOVE" should be started again to  reset  the

configuration mode (use "Options"-menu) and to permanently

store the list of selected programs into the configuration

file.

o      The  HiMOVE  memory  manager  is  now  installed  on  your

computer  and  no further user  interaction  is  necessary

during the starting phase.

3

HiMOVE (tm) Upper Memory Manager                            V2.14

3.  CONFIGURATIONPROGRAM "HISETUP.EXE"

This  program is used to install and configure the HiMOVE  memory

manager  on your computer. It also displays detailed  information

about  the  actual  memory allocation of your  computer.  If  the

configuration  file  is  not  in the  same  directory  where  the

configuration  program is located or if the  configuration   file

did not have the default filename of "HISETUP.CFG", the directory

and/or  filename  of the configuration file must be given  as  an

argument at the start of the program.

The   configuration  program  is  started  with   the   batchfile

"HIMOVE.BAT"  wich  is  located  in  the  root  directory.   This

batchfile  calls  a second batchfile with the same  name  in  the

directory where the configuration program is located. The purpose

of this procedure is to ensure that the correct arguments for the

configuration  program are used. The batchfiles will  be  created

with  the installation feature of the configuration program  (see

3.10).  This  also  circumvents the necessity  of  modifying  the

"PATH" environment variable.

If  arguments  are  entered at the start  of  "HIMOVE.BAT",  this

arguments will be passed through to HISETUP.EXE. If no  arguments

are  entered,  the arguments as defined during  the  installation

time are used as the default arguments.

3.1  Commandline Arguments

HISETUP.EXE {arg1} {arg2}

arg1:  Drive,  directory,  name and  extension  of  configuration

file.

If  no argument is given, the configuration program  scans

the directory where the HISETUP.EXE program is located for

a  file with the name "HISETUP.CFG". If this file  is  not

found, the default file name ("C:\HIMOVE\HISETUP.CFG")  is

used.

Only  parts  of the file name (drive, directory  or  name)

could be entered as first argument. For the missing  parts

of  the  full  file name, the corresponding  part  of  the

default name is used.

Please note:    A directory name must be ended with a  '\'

delimiter.

arg2:  Optional drive name of the boot drive, if it is  different

than the installation drive.

The CONFIG.SYS file will be expected in the root directory

of  this drive. If the second argument is  not  available,

the  drive of the first argument is used (or  the  default

drive "C:" if the first argument is not available).

4

HiMOVE (tm) Upper Memory Manager                            V2.14

3.1.1  Examples

"HIMOVE \SETUP\"

The  configuration  file  HISETUP.CFG  will  be  located  in  the

directory "C:\SETUP\".

"HIMOVE \HIMOVE.CFG D:"

The  full configuration file name is "C:\HIMOVE.CFG". The  memory

management   driver  HIMOVE.SYS  will  be  installed   into   the

"D:\CONFIG.SYS" file. Also a file "D:\HIMOVE.BAT" is created.

3.2  Configuration File

If  a  configuration  file  could  not  be  found,  the   default

configuration file "HISETUP.DEF" is used for the initial  program

configuration.  If any of the configuration settings is  changed,

the configuration file is rewritten with the changed parameters.

3.3  MAIN MENU

The  main menu is displayed after the start of the  configuration

program:

HiMOVE (tm) Setup V2.14 (C) Copyr.1990 C.Sitte Softwaretechnik

�����������������������������������������������������������������

�����������������������������������������������������������������

����������                                            �����������

����������  �������������� Main Menu �������������ķ  �����������

����������  � Help                              F1 �  �����������

����������  � Memory Areas                      F2 �  �����������

����������  � Options                           F3 �  �����������

����������  � Programlist                       F4 �  �����������

����������  � Resident Programs                 F5 �  �����������

����������  � DOS Memory Blocks                 F6 �  �����������

����������  � Installation of Memory Manager    F9 �  �����������

����������  � Exit                             ESC �  �����������

����������  ��������������������������������������ͼ  �����������

����������                                            �����������

�����������������������������������������������������������������

�����������������������������������������������������������������

�����������������������������������������������������������������

F1-Help  ##-Selection                                  ESC-Exit

Figure 3-1: Main Menu

By  using  the cursor up/down keys, the selection  bar  could  be

positioned  to a main menu function. The selected  menu  function

could  then  be activated by pressing the  ENTER-key.  The  first

character of a menu text or the function key on the right side of

the menu text could be used as a shortcut to directly activate  a

menu function.

5

HiMOVE (tm) Upper Memory Manager                            V2.14

3.4  MENUFUNCTION "MEMORY AREAS"

After  selection  of the menu  function  "Memory Areas  F2",  the

upper  memory  area will be analysed and then  displayed  in  the

following way:

HiMOVE (tm) Setup V2.14 (C) Copyr.1990 C.Sitte Softwaretechnik

�����������������������������������������������������������������

��������������������� Memory Areas (NEAT) �������������������ķ��

��� Area      Segment    RAM    Allocation                    ���

��� 512-528k  8000-83FF         16k DOS-RAM                   ���

��� 528-544k  8400-87FF         16k DOS-RAM                   ���

��� 544-560k  8800-8BFF         16k DOS-RAM                   ���

��� 560-576k  8C00-8FFF         16k DOS-RAM                   ���

��� 576-592k  9000-93FF         16k DOS-RAM                   ���

��� 592-608k  9400-97FF         16k DOS-RAM                   ���

��� 608-624k  9800-9BFF         16k DOS-RAM                   ���

��� 624-640k  9C00-9FFF         16k DOS-RAM                   ���

��� 640-656k  A000-A3FF <RAM1>  16k (unused)                  ���

��� 656-672k  A400-A7FF <RAM2>  16k (unused)                  ���

��� 672-688k  A800-ABFF <RAM3>  16k (unused)                  ���

��� 688-704k  AC00-AFFF <RAM4>  16k (unused)                  ���

��� 704-720k  B000-B3FF         16k Hercules                  ���

��� 720-736k  B400-B7FF         16k Hercules                  ���

��� 736-752k  B800-BBFF         16k Hercules                  ���

��� 752-768k  BC00-BFFF         16k Hercules                  ���

�������������������������������������������������������������ͼ��

�����������������������������������������������������������������

�����������������������������������������������������������������

F1-Help  ##-Selection  ENTER-Block On/Off   O-Output   ESC-Menu

Figure 3-2: Memory Areas 512kB-768kB

The  area  from 512kB till 1MB is partitioned into 32  blocks  of

16  kByte  size. The memory area screen displays a  range  of  16

blocks  out  of this area. The 'Allocation' column  displays  the

usage of any of this blocks. By using the cursor keys, the window

could be scrolled up and down.

6

HiMOVE (tm) Upper Memory Manager                            V2.14

After  pressing  the PgDn-key (Page down) the  following  display

will be shown (for example):

HiMOVE (tm) Setup V2.14 (C) Copyr.1990 C.Sitte Softwaretechnik

�����������������������������������������������������������������

���������������������� Memory Areas (NEAT) ������������������ķ��

��� Area      Segment    RAM    Allocation                    ���

��� 768-784k  C000-C3FF <RAM5>  16k (unused)                  ���

��� 784-800k  C400-C7FF <RAM6>  16k (unused)                  ���

��� 800-816k  C800-CBFF <RAM7>  16k (unused)                  ���

��� 816-832k  CC00-CFFF <RAM8>  16k (unused)                  ���

��� 832-848k  D000-D3FF         2k RAM, 14k (unused)          ���

��� 848-864k  D400-D7FF <RAM9>  16k (unused)                  ���

��� 864-880k  D800-DBFF <RAM10> 16k (unused)                  ���

��� 880-896k  DC00-DFFF         8k BIOS(Extension), 8k(unused)���

��� 896-912k  E000-E3FF <RAM11> 16k (unused)                  ���

��� 912-928k  E400-E7FF <RAM12> 16k (unused)                  ���

��� 928-944k  E800-EBFF <RAM13> 16k (unused)                  ���

��� 944-960k  EC00-EFFF <RAM14> 16k (unused)                  ���

��� 960-976k  F000-F3FF         16k BIOS-ROM                  ���

��� 976-992k  F400-F7FF         16k BIOS-ROM                  ���

��� 992-1008  F800-FBFF         16k BIOS-ROM                  ���

��� 1008-1MB  FC00-FFFF         16k BIOS-ROM                  ���

�������������������������������������������������������������ͼ��

�����������������������������������������������������������������

�����������������������������������������������������������������

F1-Help  ##-Selection  ENTER-Block On/Off   O-Output   ESC-Menu

Figure 3-3: Memory Areas 768kB-1MB

In   this  example  configuration,  the  HiMOVE  memory   manager

allocated  14  additional  blocks (RAM1-RAM14).  This  totals  to

224 kByte of additional DOS memory!

7

HiMOVE (tm) Upper Memory Manager                            V2.14

3.4.1  Allocation Texts

The following texts could appear in the "Allocation" column:

Text                   Meaning

---------------------- ------------------------------------------

xxk (unused)           This  memory area is currently not in  use

(the  memory analysis could not  determine

any usage).

NOTE: If your computer has 1 MByte of main

memory  installed, the NEAT shadow  memory

feature   could  only  be  used   if   the

additional  memory from 640kB till 1MB  is

not used as extended memory (see 5.1.1).

Extended BIOS Area     This is the extended BIOS data area.

EMS Page Frame         The EMS memory pages are mapped into  this

area.

Token-Ring-ROM         This is the ROM area of an IBM  Token-Ring

network card.

Token-Ring-RAM         This is the RAM area of an IBM  Token-Ring

network card. This RAM is only visible  if

the   Token-Ring  software   drivers   are

loaded.

xxk DOS-RAM            This memory will be used by the DOS memory

management functions. It is mainly used to

load and run application software.

CGA-Card               Color Graphics Adapter

MCGA                   Grafics-Adapter for PS/2, Model 30

EGA-Card               EGA-Card with 256 kByte RAM

EGA64k                 EGA-Card with 64 kByte RAM

EGA-MONO               EGA-Card, monochrome

Hercules               Hercules  Graphiccard

VGA-Card               Grafics-Adapter (Video-Graphics-Array)

xxk RAM                This is a RAM-Area of xx kByte size,  wich

is  not  in  use by  DOS.  This  could  be

memory,   wich  is located  on  a  network

interface card.

BIOS-ROM               This is the area of the BIOS/BASIC ROM's.

BIOS(Extension)        A BIOS extension is located in this  area.

This could be a BIOS ROM for a video  card

or a harddisk controller.

BIOS(duplicated)       Parts  of the BIOS-ROM are  duplicated  in

this area.

8

HiMOVE (tm) Upper Memory Manager                            V2.14

If this is the first configuration (no <RAMx> allocations already

made), HiMOVE allocates all the "(unused)" areas automatically as

usable  upper  memory. The allocated blocks are marked  with  the

text  <RAM1>..<RAMxx>  in  the "RAM" column of  the  memory  area

display.

A  specific  block could be selected by using the  cursor-up  and

cursor-down  keys.  The  Enter-key  toggles  a  block  allocation

between  'On' ("<RAMx>" text) and 'Off' (no text). If a  Hercules

graphics  card  is installed, four blocks could  be  appended  at

640kB  (4x16kB=64kB) to form a continuous 704 kByte  conventional

main  memory  area. By allocating memory blocks above  the  video

adapter  area,  resident  programs  (keyboard  handlers,  network

shells...)  could be loaded into these blocks, thus  freeing  the

conventional memory area.

3.4.2  Printout of memory areas

The  list  of  memory blocks could be printed or  stored  into  a

textfile. The whole list will be printed, not only the  displayed

window  area.  This function could be activated by  pressing  the

"O"-key (Output). For more information, see chapter 3.8.2.

Example Printout:

HiMOVE(tm) Setup V2.14 (C) Copyright 1990 C.Sitte Softwaretechnik

Memory Areas (NEAT)

Area      Segment    RAM    Allocation

640-656k  A000-A3FF <RAM1>  16k (unused)

656-672k  A400-A7FF <RAM2>  16k (unused)

672-688k  A800-ABFF <RAM3>  16k (unused)

688-704k  AC00-AFFF <RAM4>  16k (unused)

704-720k  B000-B3FF         16k Hercules

720-736k  B400-B7FF         16k Hercules

736-752k  B800-BBFF         16k Hercules

752-768k  BC00-BFFF         16k Hercules

768-784k  C000-C3FF <RAM5>  16k (unused)

784-800k  C400-C7FF <RAM6>  16k (unused)

800-816k  C800-CBFF <RAM7>  16k (unused)

816-832k  CC00-CFFF <RAM8>  16k (unused)

832-848k  D000-D3FF         2k RAM, 14k (unused)

848-864k  D400-D7FF <RAM9>  16k (unused)

864-880k  D800-DBFF <RAM10> 16k (unused)

880-896k  DC00-DFFF         8k BIOS(Extension), 8k(unused)

896-912k  E000-E3FF <RAM11> 16k (unused)

912-928k  E400-E7FF <RAM12> 16k (unused)

928-944k  E800-EBFF <RAM13> 16k (unused)

944-960k  EC00-EFFF <RAM14> 16k (unused)

960-976k  F000-F3FF         16k BIOS-ROM

976-992k  F400-F7FF         16k BIOS-ROM

992-1008  F800-FBFF         16k BIOS-ROM

1008-1MB  FC00-FFFF         16k BIOS-ROM

9

HiMOVE (tm) Upper Memory Manager                            V2.14

3.5  MENUFUNCTION "HELP"

This function activates the online help system. Context sensitive

help  could be displayed during the program by pressing  the  F1-

key.

HiMOVE (tm) Setup V2.14 (C) Copyr.1990 C.Sitte Softwaretechnik

�����������������������������������������������������������������

���������������������������� Help ����������������������ķ�������

�������                                                  ��������

�������               HiMOVE(tm) SETUP                   ��������

�������                                                  ��������

������� A specific menu function could be selected by    ��������

������� using the cursor keys. The selected function     ��������

������� could then be activated by pressing the          ��������

������� ENTER-Key.                                       ��������

�������                                                  ��������

������� The initial letters of the menu texts or the     ��������

������� displayed function keys could be used as a       ��������

������� short-cut to directly activate the selected      ��������

������� menu function.                                   ��������

�������                                                  ��������

�������                                                  ��������

�������                                                  ��������

��������������������������������������������������������ͼ�������

�����������������������������������������������������������������

�����������������������������������������������������������������

F1-Help  ALT:F1-Previous Help  ##-Selection            ESC-Back

Figure 3-4: Help Screen

If a helptext contains one or more subtexts, the keywords of  the

subtexts are displayed in increased intensity. A subtext could be

displayed by moving the selection bar with the cursor keys to the

keyword (selected keyword is then displayed in reverse video) and

pressing the Enter-key.

The  previously  displayed help screen(s) could be  displayed  in

reverse order by pressing ALT:F1 (ALT key in conjunction with  F1

key).  The  ESC  key removes the help  screen  and  restores  the

original screen.

10

HiMOVE (tm) Upper Memory Manager                            V2.14

Pressing the F1-key inside off the helpsystem shows a help screen

wich  contains  information  about the use of  the  help  system.

Selection  of  the  INDEX-keyword  inside  of  this  help  screen

displays the help index, wich is an overview about the  available

helptexts.

HiMOVE (tm) Setup V2.14 (C) Copyr.1990 C.Sitte Softwaretechnik

�����������������������������������������������������������������

�����������������������������������������������������������������

���������������������������� Help ����������������������ķ�������

�������                                                  ��������

�������          Welcome to the Help Function            ��������

�������                                                  ��������

�������   The cursor keys move the selection bar to the  ��������

�������   texts with increased intensity wich could be   ��������

�������   selected with the ENTER-key.                   ��������

�������   F1 calls the help-index. ALT-F1 recalls the    ��������

�������   previous help screens.                         ��������

�������                                                  ��������

�������                                                  ��������

�������                                                  ��������

�������   General   Index                                ��������

�������                                                  ��������

�������                                                  ��������

��������������������������������������������������������ͼ�������

�����������������������������������������������������������������

�����������������������������������������������������������������

F1-Help  ALT:F1-Previous Help  ##-Selection            ESC-Back

Figure 3-5: Help Screen of Helpsystem

11

HiMOVE (tm) Upper Memory Manager                            V2.14

3.6  MENUFUNCTION "OPTIONS"

After selection of the "Options", the following menu appears:

HiMOVE (tm) Setup V2.14 (C) Copyr.1990 C.Sitte Softwaretechnik

�����������������������������������������������������������������

�����������������������������������������������������������������

�����������������������������������������������������������������

�����������������������������������������������������������������

��������������                                     ��������������

��������������  ����� Installation Options ����ķ  ��������������

��������������  � Configuration Mode      = ON  �  ��������������

��������������  � Prompt on Installation  = OFF �  ��������������

��������������  � Installation Messages   = ON  �  ��������������

��������������  �������������������������������ͼ  ��������������

��������������                                     ��������������

�����������������������������������������������������������������

�����������������������������������������������������������������

�����������������������������������������������������������������

�����������������������������������������������������������������

�����������������������������������������������������������������

F1-Help  ##-Selection  ENTER-Option On/Off             ESC-Menu

Figure 3-6: Installation Options

The  selection of an installation option is done in the same  way

as in the main menu (cursor keys or first character of menu  text

line).  The  status of an option will be toggled between  ON  and

OFF.

3.6.1  OPTION 1 - "Configuration Mode"

ON:    At every start of the computer it is possible to decide if

the activation of the HiMOVE driver should be done or not,

and  wich  programs  should be loaded high  or  not.  This

functionality  is  useful during the  installation  phase,

because  in  most cases there are more than  one  restarts

necessary  to find the best combination of  programs  wich

should be loaded into the upper memory area.

OFF:   No  user  interaction  is necessary at the  start  of  the

computer  (if  option  2  is also  off).  The  loading  of

programs  wich  are  registered in  the  program  list  is

redirected to the upper memory area.

12

HiMOVE (tm) Upper Memory Manager                            V2.14

3.6.2  OPTION 2 - "Prompt on Installation"

ON:    After each start of the computer, the user must decide, if

the HiMOVE memory manager should be activated or not.

OFF:   Automatic start (if option 1 is off)

3.6.3  OPTION 3 - "Installation Messages"

ON:    Detailed message about the installation progress are shown

OFF:   No messages will be displayed

3.6.4  OPTION 4 - "Open Upper Memory"

This option is normally not visible in the Options-menu but could

be  accessed  by  pressing the  'O'-key.  The  following  message

appears:

�������������������������������������

��                                 ��

��  ������������ YES ����������ķ  ��

��  � Open Upper Memory (Y/N) ? �  ��

��  ���������������������������ͼ  ��

��                                 ��

�������������������������������������

YES:   The  upper memory is visible to the DOS memory  management

functions.  Allocation  of upper memory  is  possible  via

standard DOS functions. (YES = default)

NO:    Upper  memory  is  only available  to  the  HiMOVE  memory

manager.

Press  'Y' or 'N' to change the status of this option. Press  ESC

to remove the option display.

The option menu could be exited with the ESC-key.

13

HiMOVE (tm) Upper Memory Manager                            V2.14

3.7  MENUFUNCTION "PROGRAMLIST"

The  HiMOVE memory manager generates the programlist  during  the

configuration  phase.  Each time a program is loaded  into  upper

memory  (by  answering 'y' at the prompt), its name  is  inserted

into  the temporary programlist. When the configuration phase  is

finished  (Configuration  mode option is switched  to  OFF),  the

temporary  programlist  is written into the  configuration  file.

This  list  is  then used in the automatic mode  to  decide  wich

programs should be loaded into upper memory.

Please  note:  The HiMOVE memory manager does  not  initiate  the

loading  of  any  program. It only redirects  the  loading  of  a

program  at the time when it is started via an  operating  system

function.

The  programlist could be edited. Programnames could be  inserted

('Ins'-key)  and  deleted ('Del'-key). An existing entry  in  the

program list could also be modified ('Enter'-key).

Insertions  and  deletions in the program list changes  only  the

list of programnames in the configuration file. The HiMOVE memory

manager  does  not use the modified programlist  until  the  next

start of the computer.

HiMOVE (tm) Setup V2.14 (C) Copyr.1990 C.Sitte Softwaretechnik

�����������������������������������������������������������������

�����������������������������������������������������������������

�����������������������                   �����������������������

�����������������������  �� Programs �ķ  �����������������������

�����������������������  � MOUSE.SYS   �  �����������������������

�����������������������  � ANSI.SYS    �  �����������������������

�����������������������  � QEXT.SYS    �  �����������������������

�����������������������  � VDISK.SYS   �  �����������������������

�����������������������  � COMMAND.COM �  �����������������������

�����������������������  � KEYB.COM    �  �����������������������

�����������������������  � WATCH.COM   �  �����������������������

�����������������������  � NEWKEY.EXE  �  �����������������������

�����������������������  � DOSEDIT.COM �  �����������������������

�����������������������  � ALARM.COM   �  �����������������������

�����������������������  � IPX.COM     �  �����������������������

�����������������������  �             �  �����������������������

�����������������������  �������������ͼ  �����������������������

�����������������������                   �����������������������

�����������������������������������������������������������������

�����������������������������������������������������������������

�����������������������������������������������������������������

F1-Help ##-Selection  ENTER-Change  Ins-Insert  Del-Delete  ESC

Figure 3-7: Programlist

14

HiMOVE (tm) Upper Memory Manager                            V2.14

3.8  MENUFUNCTION "RESIDENT PROGRAMS"

This display shows all resident programs in DOS memory. The  name

of a program is displayed if possible. The free memory areas  are

also  shown.  If this list is longer than  the  available  screen

window, it could be scrolled up and down.

The  following display shows the resident programs of a  computer

WITHOUT installed HiMOVE memory manager:

HiMOVE (tm) Setup V2.14 (C) Copyr.1990 C.Sitte Softwaretechnik

�����������������������������������������������������������������

����������������������� Resident Programs �������������������ķ��

��� Segment Blocks Length Name                                ���

��� 09BB    1      19.0k  Config.sys                          ���

��� 0E7E    2       3.8k  Command.com                         ���

��� 0F66    1         48  [available]                         ���

��� 0F81    2      31.3k  CACHE.EXE ON /E/S:280 /L:1128       ���

��� 1757    2       5.4k  KEYB.COM GR 437 C:\DOS\KEYBOARD.SYS ���

��� 18AD    2       3.7k  WATCH.COM                           ���

��� 199D    2       4.3k  NEWKEY.EXE                          ���

��� 1AB8    2       2.4k  DOSEDIT.COM                         ���

��� 1B56    2        928  ALARM.COM 12:15                     ���

��� 1B92    2      15.5k  LPTX.COM -o dump.txt                ���

��� 1F78    3     514.0k  [available]                         ���

���                                                           ���

�������������������������������������������������������������ͼ��

�����������������������������������������������������������������

�����������������������������������������������������������������

F1-Help  ##-Scroll  PgUp/PgDn-Page forward/back  O-Output   ESC

Figure 3-8: Resident Programs (without HiMOVE)

It  could  be  seen from the above display,  that  the  available

conventional memory on this computer has a size of 514 kByte.

3.8.1  Description

Segment   ...  Segmentaddress of Program

Blocks    ...  Number of program memoryblocks

Length    ...  Total size of program (sum of all blocks)

Name      ...  Programname and arguments (if available)

Note:  If  a  program releases his environment block,  before  it

goes  resident,  it  is  not  possible  to  determine  the

programname and the arguments. In such a case, the segment

address  of the program will be displayed instead  of  the

name.

15

HiMOVE (tm) Upper Memory Manager                            V2.14

The  "Config.Sys"  block contains device drivers and  the  memory

areas  for the BUFFERS, FILES, FCBS and STACKS statements of  the

Config.Sys file.

After  installation  of  the HiMOVE memory  manager,  the  memory

allocation may look like the following display:

HiMOVE (tm) Setup V2.14 (C) Copyr.1990 C.Sitte Softwaretechnik

�����������������������������������������������������������������

����������������������� Resident Programs �������������������ķ��

��� Segment Blocks Length Name                                ���

��� 09BB    1       9.8k  Config.sys                          ���

��� 0C33    1        128  [available]                         ���

��� 0C3C    2      31.3k  CACHE.EXE ON /E/S:280 /L:1128       ���

��� 1412    3     623.6k  [available]                         ���

��� B000    1      64.0k  Hercules-Videoadapter               ���

��� ========== BEGIN OF UPPER MEMORY AREA AT 768k =========== ���

��� C001    1       7.6k  Device MS$MOUSE                     ���

��� C1EE    1       1.5k  Device CON                          ���

��� C252    1        544  Device QEXTXXX0                     ���

��� C275    1        736  Diskdriver                          ���

��� C2A4    2       3.8k  Command.com                         ���

��� C38C    1         48  [available]                         ���

��� C39E    2       5.4k  KEYB.COM GR 437 C:\DOS\KEYBOARD.SYS ���

��� C4FD    2       3.7k  WATCH.COM                           ���

��� C5ED    2       4.3k  NEWKEY.EXE                          ���

��� C708    2       2.4k  DOSEDIT.COM                         ���

��� C7A6    2        928  ALARM.COM 12:15                     ���

��� C7E2    2      15.5k  LPTX.COM -o dump.txt                ���

��� CBC8    1      16.8k  [available]                         ���

���                                                           ���

�������������������������������������������������������������ͼ��

�����������������������������������������������������������������

�����������������������������������������������������������������

F1-Help  ##-Scroll  PgUp/PgDn-Page forward/back  O-Output   ESC

Figure 3-9: Resident Programs (HiMOVE installed)

This gives a plus of 111 kByte in conventional memory after  four

device  drivers  and  seven  resident programs  with  a  size  of

47 kByte are moved into upper memory and the conventional  memory

area  is  increased from 640kB to 704kB by appending  a  64 kByte

block.

Please  note that the command interpreter 'Command.Com'  is  also

located in the upper memory area.

16

HiMOVE (tm) Upper Memory Manager                            V2.14

3.8.2  Print Function

The currently displayed list could be printed or saved to a file.

After pressing the O-key (Output), the following entry  field

appears:

��������� OUTPUT DISPLAY �������ķ

� PRN                            �

��������������������������������ͼ

The  default  output  channel  is the  printer  (PRN:),  but  any

filename  or  devicename  could be entered. If  the  output  file

already exists, the following question appears:

������������ OUTPUT.TXT ����������ķ

� File exists, append text (Y/N) ? �

����������������������������������ͼ

After  the  list  is  transferred, the  following  text  must  be

acknowledged with the ESC key:

������������ OUTPUT.TXT ��������ķ

� End of Text Output.  Press ESC �

��������������������������������ͼ

17

HiMOVE (tm) Upper Memory Manager                            V2.14

3.9  MENUFUNCTION "DOS MEMORY BLOCKS"

This  display  shows  the list of  memory  blocks  with  detailed

information  about  the location and length of  this  blocks  and

about  the  contents  of the accompaning  MCB's  (Memory  Control

Blocks).  If  the  list  of memory  blocks  is  longer  than  the

available screen window, it could be scrolled up and down.

The  following  display  shows the memory blocks  of  a  computer

WITHOUT installed HiMOVE memory manager:

HiMOVE (tm) Setup V2.14 (C) Copyr.1990 C.Sitte Softwaretechnik

�����������������������������������������������������������������

����������������������� DOS Memory Blocks �������������������ķ��

���ID MCB  PSP  Env. Block Length Name                        ���

��� M 09BA 0008 ---- 09BB  19.0k Config.sys                   ���

��� M 0E7D 0E7E 0F6A 0E7E   3.6k Command.com                  ���

��� M 0F65 0000 ---- 0F66     48 [available]                  ���

��� M 0F69 0E7E 0F6A 0F6A    208 Command.com (Environment)    ���

��� M 0F77 1757 0F78 0F78    128 KEYB.COM GR 437 C:\DOS\KEYBOA���

��� M 0F80 0F81 0F78 0F81   7.7k CACHE.EXE ON /E/S:280 /L:1128���

��� M 1174 0F81 0F78 1175  23.5k CACHE.EXE ON /E/S:280 /L:1128���

��� M 1756 1757 0F78 1757   5.3k KEYB.COM GR 437 C:\DOS\KEYBOA���

��� M 18AC 18B7 18AD 18AD    144 WATCH.COM (Environment)      ���

��� M 18B6 18B7 18AD 18B7   3.5k WATCH.COM                    ���

��� M 199C 19A7 199D 199D    144 NEWKEY.EXE (Environment)     ���

��� M 19A6 19A7 199D 19A7   4.2k NEWKEY.EXE                   ���

��� M 1AB7 1AC2 1AB8 1AB8    144 DOSEDIT.COM (Environment)    ���

��� M 1AC1 1AC2 1AB8 1AC2   2.2k DOSEDIT.COM                  ���

��� M 1B55 1B60 1B56 1B56    144 ALARM.COM 12:15 (Environment)���

��� M 1B5F 1B60 1B56 1B60    784 ALARM.COM 12:15              ���

��� M 1B91 1B9D 1B92 1B92    160 LPTX.COM -o dump.txt (Environ���

��� M 1B9C 1B9D 1B92 1B9D  15.4k LPTX.COM -o dump.txt         ���

��� M 1F77 1F83 1F78 1F78    160 HISETUP.EXE (Environment)    ���

��� M 1F82 1F83 1F78 1F83 118.1k HISETUP.EXE                  ���

��� Z 3D0D 0000 ---- 3D0E 395.7k [available]                  ���

��� ============== END OF DOS MEMORY AT 640k ================ ���

�������������������������������������������������������������ͼ��

�����������������������������������������������������������������

�����������������������������������������������������������������

F1-Help  ##-Scroll  PgUp/PgDn-Page forward/back  O-Output   ESC

Figure 3-10: Memory Blocks (without HiMOVE)

3.9.1  Description of header line

ID      M: Normal block, Z: Last block

MCB     Memory-Control-Block Segmentaddress

PSP     Program-Segment-Prefix Address

Env.    Environment Segmentaddress

Block   Start of memory block

Length  Length of memory block

Name    Description of memory block

18

HiMOVE (tm) Upper Memory Manager                            V2.14

After activation of the HiMOVE memory manager, the following list

will be displayed:

HiMOVE (tm) Setup V2.14 (C) Copyr.1990 C.Sitte Softwaretechnik

�����������������������������������������������������������������

����������������������� DOS Memory Blocks �������������������ķ��

���ID MCB  PSP  Env. Block Length Name                        ���

��� M 09BA 0008 ---- 09BB   9.8k Config.sys                   ���

��� M 0C32 0000 ---- 0C33    128 [available]                  ���

��� M 0C3B 0C3C 0C33 0C3C   7.7k CACHE.EXE ON /E/S:280 /L:1128���

��� M 0E2F 0C3C 0C33 0E30  23.5k CACHE.EXE ON /E/S:280 /L:1128���

��� M 1411 141D 1412 1412    160 HISETUP.EXE (Environment)    ���

��� M 141C 141D 1412 141D 118.1k HISETUP.EXE                  ���

��� M 31A7 0000 ---- 31A8 505.3k [available]                  ���

��� ========== END OF CONVENTIONAL MEMORY AT 704k =========== ���

��� M AFFF B001 ---- B000  64.0k Hercules-Videoadapter        ���

��� ========== BEGIN OF UPPER MEMORY AREA AT 768k =========== ���

��� M C000 C000 ---- C001   7.6k Device MS$MOUSE              ���

��� M C1ED C1ED ---- C1EE   1.5k Device CON                   ���

��� M C251 C251 ---- C252    544 Device QEXTXXX0              ���

��� M C274 C274 ---- C275    736 Diskdriver                   ���

��� M C2A3 C2A4 C390 C2A4   3.6k Command.com                  ���

��� M C38B 0000 ---- C38C     48 [available]                  ���

��� M C38F C2A4 C390 C390    208 Command.com (Environment)    ���

��� M C39D C3A7 C39E C39E    128 KEYB.COM GR 437 C:\DOS\KEYBOA���

��� M C3A6 C3A7 C39E C3A7   5.3k KEYB.COM GR 437 C:\DOS\KEYBOA���

��� M C4FC C507 C4FD C4FD    144 WATCH.COM (Environment)      ���

��� M C506 C507 C4FD C507   3.5k WATCH.COM                    ���

��� M C5EC C5F7 C5ED C5ED    144 NEWKEY.EXE (Environment)     ���

��� M C5F6 C5F7 C5ED C5F7   4.2k NEWKEY.EXE                   ���

��� M C707 C712 C708 C708    144 DOSEDIT.COM txt (Environment)���

��� M C711 C712 C708 C712   2.2k DOSEDIT.COM txt              ���

��� M C7A5 C7B0 C7A6 C7A6    144 ALARM.COM 12:15 (Environment)���

��� M C7AF C7B0 C7A6 C7B0    784 ALARM.COM 12:15              ���

��� M C7E1 C7ED C7E2 C7E2    160 LPTX.COM (Environment)       ���

��� M C7EC C7ED C7E2 C7ED  15.4k LPTX.COM                     ���

��� Z CBC7 0000 ---- CBC8  16.8k [available]                  ���

��� ============== END OF DOS MEMORY AT 832k ================ ���

�������������������������������������������������������������ͼ��

�����������������������������������������������������������������

�����������������������������������������������������������������

F1-Help  ##-Scroll  PgUp/PgDn-Page forward/back  O-Output   ESC

Figure 3-11: Memory Blocks (HiMOVE installed)

For the purpose of documentation and analysis, the list of memory

blocks  could  be  copied to a printer or to  a  text  file  (see

chapter 3.8.2).

19

HiMOVE (tm) Upper Memory Manager                            V2.14

3.10  MENUFUNCTION "INSTALLATION OF MEMORY MANAGER"

After  the selection of this menu function, the  installation  of

the memory manager is started with the following actions:

o  Modification   of  the  CONFIG.SYS  file.  Insertion  of   the

HIMOVE.SYS  memory manager as the first device driver  at  the

beginning  of the CONFIG.SYS File. The original file  will  be

saved as "CONFIG.BAK".

o  Creation or rewrite of the startfile "HIMOVE.BAT" in the  same

directory,  where the configuration program is  located.  This

file contains the start command for the configuration  program

"HISETUP.EXE".    This   batchfile   includes   the    correct

configuration  file  argument  and  the  optional  boot  drive

argument.  If  arguments  are  given  at  the  start  of   the

batchfile, then this arguments will be used instead.

o  Creation  of the startfile "HIMOVE.BAT" in the root  directory

of  the  same  drive, where the  CONFIG.SYS  file  is  located

(default = "C:\"). This batchfile calls the previously created

batch  file  with the same name in the  directory,  where  the

configuration program is located. Any arguments will be passed

to the second batch file.

o  When  the  following  message appears, the  computer  must  be

restarted (CTRL-ALT-DEL or reset button):

HiMOVE (tm) Setup V2.14 (C) Copyr.1990 C.Sitte Softwaretechnik

�����������������������������������������������������������������

������������������������������ Help ���������������������ķ������

��������                                                  �������

��������               END OF INSTALLATION                �������

��������                                                  �������

�������� After the installation of the memory management  �������

�������� software, the computer must be restarted (press  �������

�������� Ctrl-Alt-Del or use the reset button) to enter   �������

�������� the configuration mode.                          �������

�������� In this mode you could select on every start of  �������

�������� a program, if this program should be loaded into �������

�������� the upper memory or not. After loading of all    �������

�������� device drivers and resident programs, the        �������

�������� configuration mode should be switched off (menu  �������

�������� "Options") to make the selections permanent.     �������

��������                                                  �������

���������������������������������������������������������ͼ������

�����������������������������������������������������������������

�����������������������������������������������������������������

F1-Help  ALT:F1-Previous Help  ##-Selection            ESC-Back

Figure 3-12: End of Installation

20

HiMOVE (tm) Upper Memory Manager                            V2.14

o  After the restart, the "Configuration Mode" is on.

o  Selection of programs which should be loaded 'high' (see 4.1).

o  Termination of configuration mode (see 3.6).

NOTE:  If  the computer 'hangs' after restart, or special  add-on

cards  (network, graphics..) did not work properly,  there

is  a high possibility that there is an  address  conflict

between  the  newly allocated upper memory and  any  other

hardware in your computer.

Restart  the  computer and disable the activation  of  the

memory  manager by answering 'No' at the first  activation

question.

If the options "Configuration Mode" and "Prompt on Instal-

lation"  are  disabled, then a DOS disk must  be  used  to

reboot properly.

NOTE:  The  installation of the HiMOVE memory manager  should  be

started in the "normal" operating environment. That  means

that  all  resident programs (network,EMS,...)  should  be

loaded.

3.11  MENUFUNCTION "EXIT"

The selection of this menu function terminates the  configuration

program.

21

HiMOVE (tm) Upper Memory Manager                            V2.14

4.  MEMORY MANAGER "HIMOVE.SYS"

This device driver will be inserted by the configuration  program

at the beginning of the "CONFIG.SYS" file. The following text  is

the default text, wich will be inserted as the first line in  the

file.

"DEVICE=C:\HIMOVE\HIMOVE.SYS C:\HIMOVE\HISETUP.CFG"

If   any   arguments   are   given   at   the   start   of    the

configuration  program,  than the default statement  is  modified

according to the given arguments.

After  the call of the HIMOVE.SYS decice driver at the  start  of

the computer, the following message will be displayed:

HIMOVE.SYS V2.11 #XXXXXXXXX [XXXX] - Upper Memory Manager

(C) Copyright 1990 C.Sitte Softwaretechnik, All Rights Reserved

After  that,  the driver reads the configuration  file  (Default:

"C:\HIMOVE\HISETUP.CFG"),  wich  contains the  addresses  of  the

upper memory areas, the options and the programlist.

If  the  option "Configuration Mode" is on,  than  the  following

question appears:

>>> Configuration Mode

>>> Install Upper Memory Area (Y/N) = ?

After entering "N" (No), the boot process continues normally, and

the  memory  manager is not installed. No upper  memory  will  be

allocated  and  the  programs are loaded with  the  standard  DOS

functions.

After  entering "Y" (Yes), the allocated upper memory areas  will

be  activated  and  tested.  This  will  be  indicated  with  the

following message:

>>> Upper Memory Area activated at following address(es):

A000-AFFF  C000-CFFF

After activation of the upper memory areas, a memory test will be

done  with  this areas. If a memory error occurs,  the  following

error message will be displayed:

Memory Error at xxxx:xxxx

Instead of "xxxx:xxxx" the according "[Segment]:[Offset]"  values

of the faulty memory location are displayed. This memory area  is

automatically excluded from the following process, but should  be

22

HiMOVE (tm) Upper Memory Manager                            V2.14

disabled in the "Memory Areas" menu to avoid the error message at

the next restart (see 3.4).

After  the memory segments are activated and tested, this  memory

will  be  connected to the DOS-memory  management.  The  unusable

areas between conventional memory and upper memory will be marked

as areas wich could not be used by DOS.

4.1  Upload of resident programs

After  the  initialization of the upper memory area,  the  HiMOVE

device driver manages the loading of device drivers and  resident

programs into this area. The DOS EXEC function (Int 21,  Function

4B) is redirected into the HiMOVE device driver for this purpose.

The  following type of question appears during the  configuration

phase, when a program is loaded:

>>> Move C:\COMMAND.COM into Upper Memory:

Y)es, N)o, S)kip all ?

After  entering  "N"  (No),  the  program  will  be  loaded  into

conventional memory.

After  entering  "Y" (Yes), the program will be loaded  into  the

upper  memory  area. If there is not enough memory  to  load  the

program, the following message is shown:

>>> Cannot move C:\COMMAND.COM into Upper Memory

If   the   program  is  loaded  successfully,   the   programname

"COMMAND.COM"  will be inserted into the  temporary  programlist,

wich  is  maintained inside the memory manager.  By  calling  the

configuration program HISETUP.EXE, this list is transferred  from

the  memory  manager into the configuration  file  "HISETUP.CFG".

This  programlist  is  then used during  the  automatic  mode  to

decide,  which  programs should be loaded into the  upper  memory

area.

After  entering  "S" (Skip), no more questions  appear.  No  more

programs could be loaded into upper memory.

Note:  If other resident programs redirect the DOS EXEC function,

then  the HiMOVE memory manager is not able to upload  any

more  programs. This type of programs should be  moved  as

far as possible to the end of the start phase.

23

HiMOVE (tm) Upper Memory Manager                            V2.14

4.2  Upload of device drivers via "HILOAD.SYS"

The   HiMOVE  memory  manager  uses  a  special   device   driver

"HILOAD.SYS"  to load other device drivers into the upper  memory

area.  HILOAD.SYS  is  only a temporary device  driver,  wich  is

removed after the loading. It starts with the following message:

HILOAD.SYS - Version 2.00 - Device Driver Loader

(C) Copyright 1990 C.Sitte Softwaretechnik, All Rights Reserved

If a device driver needs more memory after its loading than it is

available, the following error message appears:

>>> Cannot start device driver

Required memory space = xxxxxx Bytes.

24

HiMOVE (tm) Upper Memory Manager                            V2.14

5.  NEAT CHIPSet

Computers with the following NEAT CHIPSet's are supported:

5.1  286 NEAT CHIPSet

CS8221 New Enhanced AT (NEAT) CHIPSet

for 80286 computers, consisting of:

82C211 CPU/BUS Controller

82C212 Page/Interleave and EMS Memory Controller

82C215 Data/Address Buffer

82C206 Integrated Peripherals Controller (IPC)

If  a  computer has 1 MByte of main memory  installed,  then  the

memory  area of 640kB-1MB could be relocated as EXTENDED  MEMORY.

In  such a case, no shadow memory could be used as  upper  memory

area  and the error message "Memory error at  xxxx:xxxx"  appears

after the start of the HiMOVE memory manager.

5.1.1  Configuration of RAM Relocation (NEAT Setup)

At  the  start of the computer, the NEAT setup could  be  started

instead of the normal boot procedure. To use shadow memory in the

addressing  range of 640kB-1MB, bit 6 of the  DRAM  configuration

register RB7 must be set to 0:

82C212  DRAM Configuration Register RB7 (6Bh)

Bit 6:  Relocate DRAM at 640kB to above 1MB

--> 0=Do not relocate RAM

1=Relocate A0000h to FFFFFh to

100000h to 15FFFFh

5.2  AT/386 CHIPSet

For 80386 Computers, consisting of:

82C301 BUS Controller

82C302 Page/Interleave Memory Controller

82A303/82A304 Address Bus Interface

82A305 Data Bus Interface

82A306 Control Signal Buffer

5.3  CS8281 NEATsx CHIPSet

For 80386SX Rechner, consisting of:

82C811 CPU/BUS Controller

82C812 Page/Interleave and EMS Memory Controller

82C215 Data/Address buffer

82C206 Integrated Peripherals Controller (IPC)

25

HiMOVE (tm) Upper Memory Manager                            V2.14

6.  QUICK INSTALLATION HELP

o  Problem:    After  loading  a  program  'high',  the  computer

hangs.

Solution:   Some programs are not designed to run in a  memory

area located above the video memory. This type  of

programs  could no be loaded 'high'. Answer NO  at

the  next  start of the computer when the question

"Move ... into upper memory?" is asked.

o  Problem:    After the start of the HIMOVE.SYS memory  manager,

"Memory error at ..." messages appear:

Solution:   The DRAM relocation of the area 640kB-1MB must  be

switched  to  OFF, to use shadow  memory  for  the

upper memory area (see 5.1.1).

o  Problem:    After the start of a graphics- or  networkprogram,

the  system  hangs or the  started  program  gives

error messages.

Solution:   The  memory  areas  wich  will  be  used  from   a

graphics-  or  network card must not be  used  for

upper memory, and should be disabled in the memory

area  screen (see 3.4). The manual of  the  add-on

card  should have the information about  the  used

memory area.

26

HiMOVE (tm) Upper Memory Manager                            V2.14

7.  To help you - help us!

If  you have questions about the configuration or  an  apparantly

unsolvable  problem with the HiMOVE memory manager  software,  we

need some information about the environment of your computer:

o  Information from the HISETUP.EXE configuration program:

- Contents of "Memory Areas"      (Function "O")

- Contents of "Resident Programs" (Function "O")

- Contents of "Memory Blocks"     (Function "O")

o  Contents of AUTOEXEC.BAT file

o  Contents of CONFIG.SYS file

o  Contents of configuration file "HISETUP.CFG"

o  Information about:

- Type of computer

- Version of operating system

- Programs (where the problem occurs)

- BIOS (manufacturer, version)

- Video adapter

- Fixed disk controller

- Add-on cards (network, graphics,...)

Send this information to a dealer wich supports the HiMOVE memory

manager or directly to us:

Mail:

C.SITTE SOFTWARETECHNIK

Postfach 42

A-5025 Salzburg/Austria

Electronic Mail:

Compuserve 73030,1070

27

HiMOVE (tm) Upper Memory Manager                            V2.14

8.  SUBJECT INDEX

2

286 NEAT CHIPSet,  25

A

Allocation Texts,  8

AT/386 CHIPSet,  25

B

BIOS(duplicated),  8

BIOS(Extension),  8

BIOS-ROM,  8

BUFFERS,  16

C

CGA-Card,  8

Commandline Arguments,  4

CONFIG.SYS,  20,  22

Configuration File,  5

Configuration Mode,  12

Configuration of RAM Relocation (NEAT Setup),  25

CONFIGURATIONPROGRAM "HISETUP.EXE",  4

CS8281 NEATsx CHIPSet,  25

D

DEFINITION OF TERMS,  1

Description,  15

Description of header line,  18

Device driver,  24

Display memory blocks,  18

DOS MEMORY BLOCKS,  18

DOS-RAM,  8

E

EGA-Card,  8

EGA-MONO,  8

EGA64k,  8

EMS Page Frame,  8

Examples,  5

EXEC,  23

EXIT,  21

Extended BIOS Area,  8

28

HiMOVE (tm) Upper Memory Manager                            V2.14

F

F1,  3,  10,  11

FCBS,  16

FILES,  16

FIRST MEGABYTE,  1

FUTURE,  3

G

GENERAL,  1

H

Hercules,  8

HiMOVE CONCEPT,  2

HIMOVE.BAT,  4,  20

HIMOVE.SYS,  20

HISETUP.CFG,  4,  22

HISETUP.EXE,  4

I

INSTALLATION,  2,  20

Installation Messages,  13

Installation Procedure,  3

L

LICENSE,  2

M

MAIN MENU,  5

MCGA,  8

Memory Areas,  6

Memory Error,  22

Memory error at,  26

MEMORY MANAGER "HIMOVE.SYS",  22

MENUFUNCTION "DOS MEMORY BLOCKS",  18

MENUFUNCTION "EXIT",  21

MENUFUNCTION "HELP",  10

MENUFUNCTION "INSTALLATION OF MEMORY MANAGER",  20

MENUFUNCTION "MEMORY AREAS",  6

MENUFUNCTION "OPTIONS",  12

MENUFUNCTION "PROGRAMLIST",  14

MENUFUNCTION "RESIDENT PROGRAMS",  15

N

NEAT CHIPSet,  25

NEW ENHANCED AT CHIP SET (NEAT),  1

29

HiMOVE (tm) Upper Memory Manager                            V2.14

O

OPTION 1 - "Configuration Mode",  12

OPTION 2 - "Prompt on Installation",  13

OPTION 3 - "Installation Messages",  13

OPTION 4 - "Open Upper Memory",  13

OPTIONS,  12

Order Information,  4

P

PATH,  4

Print Function,  17

Printout of memory areas,  9

Programlist,  14

Prompt on Installation,  13

Q

QUICK INSTALLATION HELP,  26

R

RAM,  8

REMARKS,  1

Resident Programs,  15

S

SHAREWARE,  2

Shareware Copy from BBS or Disk Vendor,  2

STACKS,  16

SUBJECT INDEX,  28

T

To help you - help us!,  27

Token-Ring-RAM,  8

Token-Ring-ROM,  8

U

Upload of device drivers via "HILOAD.SYS",  24

Upload of resident programs,  23

V

VGA-Card,  8

W

WARRANTY,  2

30