Readme file for RomLESS FDSCSI v1.00 Beta-01

Date: 07/22/1993

=================

1.0 Introduction

=================

This readme file describes the RomLESS FDSCSI v1.00. It provides

features and installation instructions.

==================

2.0 Features

==================

o  The RomLESS FDSCSI will allow DOS to support a SCSI fixed disk

without a ROM BIOS installed on the SCSI controller.

o  This utility will support up to 7 SCSI fixed disk drives.

o  This utility will support any capacity disk drive up to 8 GBytes.

o  The drive must be the secondary drive installed.  This utility

will not support a drive as the primary bootable drive.

o  This utility supports MS/PC DOS 3.x through 6.0.

====================

3.0 Contents

====================

3.1 SETUP.EXE

This performs a optional low-level format on the SCSI drive and

also can perform a comprehensive SURFACE ANALYSIS for media

defect detection.  Setup will update the defect list built in the

drive if any defects are found.

3.2 FDFDISK.EXE

This creates the disk partition on the SCSI disk drive.  This

program is required for FDDEVICE.SYS to support the disk drive.

This program should be used instead of DOS's FDISK.COM.

3.3 FDFORMAT.EXE

This performs the DOS format and copies over the system files

to the SCSI drive.  This program can also perform a disk verification

for bad tracks.

3.4 FDDEVICE.SYS

This is the driver loaded in memory which will allow DOS to access

any SCSI drive attached to the Future Domain SCSI controller.  This

file should reside in the CONFIG.SYS file as:

DEVICE=C:\%PATH%\FDDEVICE.SYS

3.5 DCAM950.EXE and DCAM1800.EXE

These are the SCSI controller drivers required to interface DOS to

the SCSI drives.

DCAM950.EXE is for the TMC-8xxM SCSI controllers.

DCAM1800.EXE is for the TMC-16xx SCSI controllers and

the MCS-600/700 Micro Channel SCSI controllers

These files must be installed prior to FDBIOS.SYS and FDDEVICE.SYS.

Refere to the installation steps for installation instructions.

3.6 FDBIOS.SYS

This driver is the INT13h interface.  It will route any INT13h

calls to the appropriate SCSI disk supported.

==================

4.0 Installation

==================

1. Boot from the primary bootable drive (DRIVE C:).

2. Insert the RomLESS FDSCSI v1.00 distribution diskette in the

A: floppy drive.

3. Change to the A: floppy drive and run SETUP.EXE

[C:\] A: <Enter>

[A:\] SETUP <Enter>

4. Run low level format on SCSI drive(s).

5. Run FDFDISK.EXE and create partition on SCSI drive(s).

6. Run FDFORMAT.EXE and high-level format the SCSI drive(s).

7. Remove the RomLESS FDSCSI v1.00 diskette from the A: floppy

drive and reboot your system <Ctrl>+<Alt>+<Del>

8. Re-insert the RomLESS FDSCSI v1.00 diskette in the A: floppy

drive.

9. Change to the A: floppy drive and run SETUP.EXE

[C:\] A: <Enter>

[A:\] SETUP <Enter>

10. Copy FDDEVICE.SYS, DCAMxxxx.EXE, and FDBIOS.SYS over to

the C: drive.

11. Edit the CONFIG.SYS file and load the drivers in the

following order.

DEVICE=C:\DCAM950 or DCAM1800.EXE

DEVICE=C:\FDBIOS.SYS

DEVICE=C:\FDDEVICE.SYS

12. Installation is complete.  Reboot the system.