UltraStor Corporation

The Finest in Storage Controller Solutions

Installation Instructions for the ULTRA 14F with Novell NetWare 3.11

With More than 16MB of System Memory

-----------------------------------------------------------------------------

The following is the procedure to support more than 16MB of memory in a

system operating under Novell NetWare v3.11

1.  Include "Set Reserved Buffers Below 16Meg=-xx" as the first line in the

STARTUP.NCF, where xx indicates the number of buffers to be reserved,

ranging from 8 minimum to 200 maximum.  Generally, each drive will require

20 buffers to be reserved.

Example, to allocate buffers for 2 hard drives, include the following on

the first line in the STARTUP.NCF.

Set Reserved Buffers Below 16 Meg=40

2.  Add "Set Auto Register Memory Above 16 Megabytes=Off" in the STARTUP.NCF

after the Reserved Buffer statement.

3.  Load U14_31X.DSK with /b at the console or include it in the STARTUP.NCF

to specify access for more than 16MB.

:LOAD U14_31X /b

4.  Add "Register Memory ssssllll" in the AUTOEXEC.NCF after the lines for

the file server name and the IPX internal NetWare number to add memory

above 16MB, where "ssss" means start of memory and "llll" shows the length

in hex values.

Example to add 16MB of memory above 16MB, type:

:Register Memory 1000000 1000000

at the system console, or include the above statement in the AUTOEXEC.NCF

file.