E2000plus-series Ethernet Adapters

Installation file for

\WFW311 directory

============================================================================

************************************************************

* NDIS Driver Installation for Windows for Workgroups 3.11 *

************************************************************

1. To install NDIS drivers in Windows for Workgroups, user

should be in the Windows environment first and choose the Network

Setup icon.

2. Click on "Drivers" in the Network Setup window.

3. Choose the "Add Adapter" item in network driver.

4. When the new menu is displayed, choose the "Unlisted or Updated

Network Adapter" item for the new driver.

5. There is a box for entering the device and directory. Key in

"A:\WFW311" <enter>.

6. Input the correct values for IRQ channel and I/O address. These

two values must agree with adapter's actual parameters. Use the

View Configuration function in the SETUP.EXE program to see them.

For the Others as Required input, nothing needs to be entered.

7. Click the Driver Setup icon in the Network Driver window. Set the driver

type to be "real mode", "real and enhanced mode", or "enhanced

mode" NDIS driver, leave Windows for Workgroups, and reboot.