Font Installation Utility ( Version 1.0) for Lotus 123

------------------------------------------------------

Release 2.2 and Lotus Symphony Release 2.2.

-------------------------------------------

24th April 1991

DESCRIPTION

------------

The program sw2finst.exe is a utility which modifies Lotus 2.2 text

printer drivers so that they automatically select a particular font.

This can either be one of the resident fonts or a font on an installed

font card. It does this by inserting an escape sequence into the printer

driver. This escape sequence is taken from a text file ( typically

with the extension .DAT ).

The program has been designed to read a normal text file containing

the escape sequences used to select fonts. It is possible to support

new fonts by simply editing this file using any standard editor.

USAGE

-----

The supplied floppy disk has two .DAT files

CANBELQ.DAT  for Canon BJ printers in Epson LQ mode

CANBPRO.DAT  for Canon BJ printers in ProPrinter mode

The supported printers in the Canon BJ-Series are the  BJ-300

and BJ-330.  The BJ-300 and the BJ-330 can work in two modes, IBM Pro-

Printer mode and Epson LQ mode. Dip switches must be set to select one

of these modes and details of how to do this are in the User Manual.

The Canon BJ-10e does not support font cards. The utility can be

used to select resident for for the BJ-10e in ProPrinter mode.

Copy these files from the floppy disk to your Lotus directory

using the batch file CPYFINST.BAT

SW2FINST.EXE

SW2FINST.DOC

CANBELQ.DAT

CANBPRO.DAT

CANBELQ.DRV

CANBPRO.DRV

ELQFONT.BAT

PROFONT.BAT

E.g. to install the font installer on C:\123 from floppy A:

Place the driver pack disk in A:

Type

A:

Then press ENTER to change to drive A:

Type

CPYFINST C:\123

Then press ENTER to copy all font installer files to C:\123

Before using the utility change to your Lotus directory.

E.g. if you have Lotus 123 on drive C: in the directory \123

Type

C:

Then press ENTER to change to drive C:

Type

CD \123

Then press ENTER to change to directory \123

To run the font installation program :

Type :

sw2finst data file name  driver name

Then press ENTER

E.g. if the datafile is  CANBELQ.DAT  and the driver is CANBELQ.DRV

Type :

sw2finst CANBELQ.DAT CANBELQ.DRV

Then press ENTER

Alternatively you can use one of the supplied batch files.

ELQFONT.BAT  to install a font in the Canon Epson LQ driver  CANBELQ.DRV

PROFONT.BAT  to install a font in the Canon ProPrinter driver CANBPRO.DRV

When it starts program will display some information taken from the

data file.

Press a key to continue.

The program then displays a list of all font cards it found in the

data file ( there may only be one ).

Select a font card by typing its number and pressing ENTER.

The program then lists any fonts supported by the font card.

Select a font by typing its number and pressing ENTER.

The program then modifies the driver so that it will select the chosen

font.

Before you can use the font you must run the Lotus INSTALL program

and use this to add and save the new driver and select the text printer

driver. See your Lotus documentation for details of this but briefly

you need to do the following :

1. Change to the Lotus application directory

2. Type  :  INSTALL ENTER

3. Press ENTER to continue with installation

4. Select 'Change Selected Equipment'  ENTER

5. Select 'Text Printer'

6. Select 'Canon'

7. Select one of the available printers

8. Select 'Save Changes'

9. Select 'Yes'

10. Select 'Exit Install Program'

11. Select 'Yes'

The Lotus application will have now registered the new Canon

printer drivers.

Diagrammatically the options are :

Advanced Options :

Add new drivers to library

Main Menu        :

Change selected equipment :

Text Printer(s) :

|>  Canon  :

BJ-Series ( IBM Pro Mode)

Save Changes

Exit

Naturally to be able to use the font the appropriate font card must

be installed in your printer.

DETAILS OF TEXT FILE LAYOUT

The text file is organised as a header followed by information about

any supported font cards. The supplied .DAT files provide examples.

COMMENTS

Any line in the text file which begins with a '#' is treated as a

comment

E.g.

# This line is a comment and will be ignored.

HEADER FORMAT

All non-comment lines preceding the name of the first font card are

treated as a header and displayed on-screen when the font installation

program is run. Typically this could include details of the supported

printers. For example :

------------------------------------------------------------------

The following printers are supported :

Canon Bubble-Jet Printer BJ-300 ( Epson LQ mode)

Canon Bubble-Jet Printer BJ-330 ( Epson LQ mode)

-------------------------------------------------------------------

FONT CARDS Any line which begins with two '>>' symbols is treated as

the name of a font card. E.g. the line :

>> BJFC-301 GENERAL

Is used to indicate the BJFC-301 GENERAL font card for the Canon BJ-300

and BJ-330 printers.

FONT NAMES The lines following a font card name give details of the

supported fonts. These lines must begin with a text string enclosed

in double quotes and end with a sequence of decimal numbers which

describes the escape sequence used to select a font. Each decimal

number must be between 0 and 255. The first decimal number is a count

indicating the numbers of numbers following. The decimal numbers must

be separated by spaces or TABS.

E.g. The line :

"Courier            10        " 9 27 82 0 18 27 107 2 27 80

Shows that to select the font "Courier        10   " the nine numbers

27,82,0,18,27,107,2,27 and 80 are needed.

These numbers (in Epson LQ mode for the Canon ) represent the

Escape sequence :

ESC R 0          Reset TABS to defaults

DC2              Cancel condensed mode

ESC k 2          Select Courier typestyle

ESC P            Select 10 cpi

To add information for other printers see the relevant printer manuals

for Escape sequences.