(Comp.sys.handhelds)

Item: 904 by ard at siva.bristol.ac.uk

Author: [Tony Duell]

Subj: HP48 digital logic simulator

Date: Mon Oct 15 1990 08:05

Here's a digital circuit simulator for the HP48SX. It has 8 internal

primatives: AND, OR, XOR, NOT, BUF (buffer), HI (node tied to logic 1),

LO (node tied to logic 0) and CLK (clock generator of definable high and

low times). It also supports nested macros defined from these elements.

A circuit is entered as a list of lists. Each sublist defines one gate (or

macro-circuit) in the design. The format of the sublist is as follows :

{"NAME" I1 I2 I3..}

Where NAME is either one of the primatives above or the variable name where the

macro is stored entered as a string. I1 I2 etc are the names of the nodes in

your circuit, and must be valid HP48 name objects (or in the case of "CLK", the

first 2 arguments are integers). For example:

{"AND" A1 A2 Both}  is the representation of the circuit :

A1----------!\

! )--------- Both

A2----------!/

In the primatives, the inputs are always given first, and I suggest you do

the same in macros (see below).

2 - input gates

---------------

There are 3 types of 2-input gate defined, AND, OR and XOR. The formats are

list format          equivalent expression (which you don't enter)

{"AND" A1 A2 Q}       Q= A1 AND A2

{"OR" A1 A2 Q}        Q= A1 OR A2

{"XOR" A1 A2 Q}       Q= A1 XOR A2

1 - input gates

---------------

There are 2 types of 1-input gate, NOT and Buffer:

list format        equivalent expression

{"NOT" A B}         B=NOT A

{"BUF" A B}         B=A

Tied nodes

----------

A node can also be defined as permanently high or low :

list format    equivalent expression

{"HI" A}        A=1

{"LO" A}        A=0

Clock Generators.

The high and low times of the clock generator are defined in terms of

time-steps which are defined as 1 horizontal pixel on the final timing diagram.

Note that all gates have a propagation delay of one time-step. The format for

defining a clock generator is :

{"CLK" hi_time lo_time Output}

Where hi_time and lo_time are real numbers giving the number of time-steps that

the output will be high or low respectively and Output is the node name for the

output.

e.g. :

{"CLK" 4 4 A} defines that node A will be low for 4 time-steps, high for 4

time-steps, low for 4 time-steps etc.

A circuit is defined by creating a list of these sub-lists e.g.:

{{"CLK" 2 2 A} {"CLK" 4 4 B} {"AND" A B C}}

The only commands you need to know to run the simulator are as follows:

COMPILE

-------

Takes the variable name where a circuit is stored in top of stack, and sets up

the internal variables to represent that circuit. Use this before attempting to

simulate

NODES

-----

Takes a list of nodes which are to appear in the final timing diagram in top of

stack, and stores it in the internal variable OUT.L

ALL

---

sets up OUT.L to contain _ALL_ nodes in the circuit (including the internal

ones in macros). Only useful for simple circuits.

SIM

---

Takes a length of simualtion in top of stack (a real number, max 100) and plots

the timing diagram (in PICT) for the given circuit. After it has finished,

press any key (except ATTN) to exit and restore the stack display

Example :

Given the circuit defined above in top of stack, store it in 'CSH'

then execute the following

'CSH' COMPILE            set up this circuit

ALL                      we'll show all nodes

100 SIM                  draw a timing diagram.

MACROS

------

A macro-circuit is a bit like a subroutine - it's a part of a circuit (e.g. a

half-adder or a flip-flop) which is used several times. For an example, see the

variable HADD in the directory which contains the macro that defines a half

adder.

Each macro definition has the form

{{External list} {{GATE1}{GATE2}...}}

where {{GATE1}{GATE2}...} is the circuit description of the macro as defined

above, and {External list} is the list of external connections.

e.g. the 2-input multiplexor (variable MUX) in the directory

Circuit

!\

SEL --+----! >o------------!\

!    !/              ! )--!

!             A------!/   ------\----\

!                                )    )-------- Y

!             B------!\   ------/----/

!                    ! )--!

!--------------------!/

Now, the external connections are SEL, A, B and Y

!\     NS

SEL --+----! >o------------!\

!    !/              ! )--! YA

!             A------!/   ------\----\

!                                )    )-------- Y

!             B------!\   ------/----/

!                    ! )--! YB

!--------------------!/

I've now labeled the internal nodes (NS, YA, YB)

Thus, we get the definition:

{{SEL A B Y}{{"NOT" SEL NS} {"AND" A NS YA} {"AND" B SEL YB} {"OR" YA YB Y}}}

If this is stored in the variable MUX , the element

{"MUX" X Y Z GO} will be valid, with the correspondence:

X -> SEL

Y -> A

Z -> B

GO -> Y

circuits (and other macros) can be defined in terms of MUX.

Example : the full adder

------------------------

I have included a simple example to demonstrate the features of this logic

simulator : the single-bit full adder.

In the variable HADD is the macro for a half - adder, whereas in FADD, I have

given the macro for a full-adder, defined in terms of FADD

In the variable DEMO, I have defined a simple circuit with one full-adder, with

its 3 input driven from clock-generators in the ratio 1:2:4. Here's how to run

the demonstration:

'DEMO' COMPILE      define the circuit

{M N P S C} NODES   only display the inputs and outputs

100 SIM             display the timing diagram (after it's finished, press any

key to exit)

I am quite willing to answer questions about how this program works, but as

it's likely to be quite long, I'll only post it to the net if there is

sufficient interest.....

-Tony Duell

ARD @ UK.AC.BRIS.PVA  (JANET)

ARD @ SIVA.BRIS.AC.UK (BITNET)

(This program and its documentation are public domain. They may be freely

copied, used and published in user-group newsletters provided that my name

remains attached to them. They may not be sold for profit)