#

File: panasoni.info,  Node: Top,  Prev: (orion.info),  Next: (philips.info),  Up: (remotes.info)

******************************************************************************

PANASONIC

******************************************************************************

* Menu:

* Technical info::

* TV::

* Video::

* Amp::

* CD::

* Tape::

* Tuner::

#

File: panasoni.info,  Node: Technical info,  Prev: Top,  Next: TV,  Up: Top

******************************************************************************

TECHNICAL INFO

******************************************************************************

Code length:            48 bits

Carrier:                32kHz

+--------------------+

Header:      |                    |

+                    +--------+..

10T            4T

+--+

1 is coded:  |  |

..+  +------+..

T    3T

+--+

0 is coded:  |  |

..+  +--+..

T   T

+--+

|  |

After the last bit a ( ..+  +  ) is transmitted to terminate the last bit.

T

Space between transmissions 190T.

T = 400us approx

For more information please read the documentation.

#

File: panasoni.info,  Node: TV,  Prev: Technical info,  Next: Video,  Up: Top

******************************************************************************

TV

******************************************************************************

GROUP   #40040100

Code    Function

----    --------

#BCBD   Power

#4C4D   Mute

#DCDD   -/--

#0AC5   C

#9899   0

#0809   1

#8889   2

#4849   3

#C8C9   4

#2829   5

#A8A9   6

#6869   7

#E8E9   8

#1819   9

#

File: panasoni.info,  Node: Video,  Prev: TV,  Next: Amp,  Up: Top

******************************************************************************

VIDEO

******************************************************************************

Group:	#40040900

Code	Function

----	 --------

#BCB5	VTR

#5059	Play

#0009	Stop

#4049	Rew

#C0C9	FF

#6069	Pause

#3039	Still adv

#1019	Rec

#F0F9	Slow

#7178	Slow+

#F1F8	Slow-

#0801	1

#8881	2

#4841	3

#C8C1	4

#2821	5

#A8A1	6

#6861	7

#E8E1	8

#1811	9

#9891	0

#DCD5	-/--

#6A63	Clock/Counter

#6C65	VTR/TV

#030A	Input select

#2A23	Reset

#8980	Monitor

#222B	Time search

#8D48	Tracking+

#4D44	Tracking-

Codes taken from a NV-FS100EG by Frank Heepmann.

They are tested on his VCR (German version).

#2C25	channel +

#ACA5	channel -

#CAC3	memory/repeat/search

#AAA3	remaining time

#CCC5	audio out

#0900	blank search

#2D24	timer

#020B	index		\  For the VISS (VHS index

#828B	address		| search system) and VASS

#D2DB	write		| (VHS address search

#323B	erase		| system) features.

#424B	renumber	/

#8188	jog/shuttle on	\  These codes allow to

#3039	jog >		| emulate the jog/shuttle

#BOB9	jog <		| button on the RC.

#1118	shuttle <<< 	|

#6168	shuttle << 	| > = forward

#2128	shuttle <	| < = backwards

#4148	shuttle <|	|

#C1C8	shuttle |> 	| examples:

#A1A8	shuttle >	| |>  = slow motion forward

#E1E8   shuttle >>	| <<< = very fast backwards

#9198   shuttle >>>	/

GROUP:	#40040980

Code	Function

----	--------

#8009	check timer

#50D9	tape speed (sp/lp)

#0881	language select	\  These codes are only

#028B	text on		| successful if the VPT

#C24B	text off 		| (timer programming by

#22AB	index		| videotext) option is

#A22B	f/t/b (text size)	| installed. The module in

#62EB	reveal		| my VCR is called VW-VPT1.

#E26B	hold		| It also allows TOP (table

#52DB	list		| of pages) videotext, a very

#0A83	red		| fast page access system.

#8A03	green		| (videotext = teletext)

#4AC3	yellow		|

#CA43	blue		| In Germany the NV-FS100EG

#72FB	top/memory	| was Panasonic's one and only

#B23B	page preset	| VCR model with this option

#F27B	vpt		/  (unforunately).

---------------------------------

#

File: panasoni.info,  Node: Amp,  Prev: Video,  Next: CD,  Up: Top

******************************************************************************

AMP

******************************************************************************

GROUP   #40040543

Code	Function

----	--------

#FDBB	On/Off

#0D4B	Vol+

#8DCB	Vol-

#3D7B	Sleep

#F17B	Display

#

File: panasoni.info,  Node: CD,  Prev: Amp,  Next: Tape,  Up: Top

******************************************************************************

CD

******************************************************************************

GROUP   #40040543 (Same as amp)

Code      Function

----      --------

#E1A7     Play

#0147     Stop

#2167     Rew

#A1E7     FF

#D197     Program

#B1F7     Cancel

#4107     Random

#81C7     Repeat

#3094     1

#89CF     2

#490F     3

#C98F     4

#296F     5

#A9EF     6

#692F     7

#E9AF     8

#195F     9

#99DF     0

#F9BF     +10

#

File: panasoni.info,  Node: Tape,  Prev: CD,  Next: Tuner,  Up: Top

******************************************************************************

TAPE

******************************************************************************

GROUP   #40040543 (Same as amp)

Code      Function

----      --------

#6523     Play

#A5E3     RewPlay

#0543     Stop

#4503     Rew

#C583     FF

#7533     Reverse mode

#F5B3     Counter reset

#B5F3     Deck 1/2

#

File: panasoni.info,  Node: Tuner,  Prev: Tape,  Up: Top

******************************************************************************

TUNER

******************************************************************************

GROUP   #40040543 (Same as amp)

Code      Function

----      --------

#2761     Fm mode

#1D5B     Tune/Band

#3094     1

#89CF     2

#490F     3

#C98F     4

#296F     5

#A9EF     6

#692F     7

#E9AF     8

#195F     9

#99DF     0