Wssindex/Extended Torture Test

A user on the BBS asked whether Wssindex/Extended could handle

indexing 10 CD-ROMs full of public domain and shareware files.

My initial response  was that it should be able  to do so, but

to my knowledge, no one had pushed it that hard.  I decided to

do  some tests.  After seeing the results, I made some changes

to improve the performance when swapping is necessary.

Initial test configuration:

33 MHz 486

16 MB RAM with 2 MB RAM disk/cache, so effectively 14 MB RAM

Mitsumi single speed CD-ROM drive

MS-DOS 5

WSGCC532.EXE, the latest released Wssindex/Extended for 386+

at the time the tests were run

Test date June 5, 1994

Test disks (9 total)

Hobbes OS/2 3/94

Simtel MS-DOS

12/92 edition

10/93 edition

JCSM Shareware Collection

DOS 3/94

Windows/OS2 3/94

DOS/OS2 11/93

Windows/NT 11/93

Chestnut

Deathstar Arcade  Battles (warning XYPHR2 on  this disk is

infected with  the power pump virus,  detected by F-PROT

and recent versions of SCAN)

Our Solar System

Indexing options

Archive extraction enabled (all types)

Self-extracting file extraction disabled

Indexing  time was not carefully measured.  It is dominated by

the speed  of the  CD-ROM.  With  my slow drive,  times ranged

from 10  minutes to nearly an hour.  This was definitely a "go

do  something else and come back when it's finished" test (but

only because archive extraction was  enabled).  With 600 MB on

some of the disks  and 150 KB/sec  data rate, 4000 seconds  (a

little  more than  an hour)  would be  required to  read every

byte.  Extracting info  from compressed files is not  quite as

bad as  having to read every byte, but  it is close.  The data

rate from a 1.2 MB floppy is 45 MB/sec; even  double or triple

speed CD-ROMs are slow for the amount of data they hold.

First  timing  test, after  indexing  252,709  files (at  this

point, disk swapping was not needed)

select all files - a few seconds

sort, keys  1,2,3 (file  name,  extension, disk  name) -  56

seconds

Second timing test, after  indexing 363,845 files (now paging,

swap file on a disk compressed by Stacker for OS/2 and DOS)

select all files - 100 seconds

sort, keys 1,2,3 - 44 min, 19 seconds

Move swap file to uncompressed disk

database load time from  compressed disk (11.4 MB) -  1 min,

43 seconds

select all files - 73 seconds

sort, keys 1,2,3 - 19 min, 50 seconds

Preliminary conclusions

1.  Wssindex/Extended  does not  choke  on a  large number  of

files.

2.  Performance  degrades dramatically  when disk  swapping is

necessary.   (To  estimate memory  requirements,  allow 42

bytes per file plus space for comments and categories.)

3.  Swapping  to  a  compressed   disk  causes  an  additional

degradation of more than a factor of two.

4.  Huge databases  require a significant amount  of free disk

space for paging and for database storage.

Wssindex/Extended version 5.34 added  a merge sort option: the

list of selected files is split in half, the halves are sorted

separately  and then merged.   The idea is  that if sufficient

memory is available  to hold half the files, no paging will be

needed while sorting.   The  disk activity light  serves as  a

monitor of the sort  progress.  There is a period  of activity

as the first half  of the data is  swapped in, then a  lull as

the  data is sorted  in memory.   This repeats for  the second

half, then  the disk is  continuously active during  the merge

phase.

Tests  were  done  under   (a) DOS,  (b) Windows  3.1  with  a

temporary  swap  file,   (c) Windows  3.1  with   the  largest

permanent  swap  file it  claimed  it would  use  (13 MB), and

(d) Windows 3.1 with a 20MB swap  file.  The first test with a

permanent swap file  ran out  of memory when  it attempted  to

allocate the additional memory needed to do the merge.

A  brief  tutorial  on  how  virtual  memory   works  may  aid

interpretation of  the results.  Disk swapping  is hidden from

Wssindex; physical  and virtual memory  are indistinguishable.

When  Wssindex attempts  to  reference memory  which has  been

swapped to disk, a hardware page fault is generated.  The page

fault  is serviced  by  whichever host  is providing  the DPMI

services.    This  might be  the  GO32  extender  built in  to

Wssindex, or it  might be 386MAX,  OS/2, QDPMI, Windows,  etc.

The paging routine swaps a page  of memory out to disk to make

room  for the  page which  needs to  be swapped  in.   But the

paging  routine doesn't know what pages are going to be needed

again, so  it may  make a  bad choice and  swap out  something

which will needed soon.  Thus, the paging algorithm can have a

big effect on performance.

Test date Oct 8-9, 1994, pre-release WSGCC534.EXE

sort          DOS     Win 3.1,      Win 3.1,      Win 3.1,

363,845               temporary    13MB perm      20MB perm

files                 swap file    swap file      swap file

Allocate       6         244           77            77

memory

Load           83        237           85            85

database

Select all     69        254           70            58

files

Sort first     82        141           83            67

half

Sort           91        126         out of          80

second                               memory

half

Merge         422       4204          ---           1135

Total sort    596       4471          ---           1282

time

(Times are rounded to the nearest  second.  Total time may not

equal sum  of  sort and  merge  times  due to  rounding.    On

repeated runs, times vary by a few percent.)

Conclusions

1.  Overall, the fastest sorts are under DOS.

2.  Windows actually  sorts the  two halves faster,  but falls

down on the merge.

3.  Performance under  Windows with  a temporary swap  file is

horrible.

4.  Windows  will use  a larger permanent  swap file  than the

setup routine  claims, and increasing the  size beyond the

minimum required may improve performance.

5.  The reason  for the decrease  in load time under  DOS from

103  seconds in  the early  tests  to 83  seconds here  is

unknown, but may reflect defragmenting of the disk holding

the database.