------------------------------------------------------

DDT(8080)/ICUAP

Universidad Autonoma de Puebla

August 30, 1983

Harold  V.  McIntosh

------------------------------------------------------

A = assemble

B = bitmap

D = display

F = fill

G = go

H = hex sum, difference

I = initialize CCP command line

L = list program

M = move

R = read file

S = store

T = type registers

V = single step w/ call as 1 step

(A) Assemble 8080 code. <nAx> will assemble n lines

of code beginning at address x. Defaults are x=lorg,

while no limit is set if n is missing or zero.

(B)  Generate a bitmap of all bytes in the range

100H - (pmax) which could possibly be the address

of an instruction.

(D) Display the memory in groups of 16 bytes, both as

pairs of hexadecimal nibbles and as ASCII characters,

when the latter is appropriate. The command <nDa,b>

will display n lines, or else the interval between

hexadecimal addresses a and b; the interval takes

precedence but if a or b or both are null, the line

count helps determine the interval. When a is null the

display will usually continue from where it last ended,

but nD,b will display b-n up to b. In general:

nDa,b	a through b-1, n lines of 16 bytes

Da,b   a through b-1, DP lines of 16 bytes

nDa,	a through a+n

nD,b	b-n through b

nD,	PC through PC+n

Da,    a to the end

Da	just line containing a

D,b	PC through b

D,	PC to the end

D	just line beginning at PC

(F) Fill an interval. <Fx,y,a> will fill the interval

from x to y with the hexadecimal value a.

(G)  Go to the program for execution. The form <nG,a,b,c>

allows the insertion of up to two breakpoints, b and c,

before jumping to the address a. If a count n is also

included, the breakpoints will be traversed n times

before control is t returned to DDT. If the field a is

null, the program will continue from the point where

control was surrendered to DDT (or 100H if it is an

initial call.

(H) Display a hexadecimal sum and difference. <Hx,y>

will result in typing '+'x+y '-'x-y. Mainly used to

help patch CP/M during system generation.

(I) Input a command line, according to the same

criteria used by CCP; in other words a parameter

list can follow the file name.

(L) List the memory as 8080 operation codes. The

command <nLa,b> will list n lines of code between

the hexadecimal addresses a and b, with the interval

taking precedence over the number of lines, unless

either a or b is missing.

(M) Move part of the memory from one place to another.

<Mx,y,z> will move the interval x,y to begin at z. The

direction of movement compensates for possible overlap.

(R) Read into memory the file specified by TFCB.

It was probably defined by a preceding I command,

but might still be the program defined in the DDT

command line. <Rx> will start loading at TORG+x.

(S) Store hexadecimal bytes. <nSx> will store

n bytes starting at address x, read one by one

from the console. Missing n means indefinite

repetition, missing x means TORG. Confirmation

and progression is automatic while hexadecimal

bytes are being received; however, for

-  go back one byte,

'x  deposit ASCII x,

^x  deposit ctl x,

CR  go on to next byte.

Any other input terminates the cycle.

(T)  Type the register status line while single-stepping

through the program. <nT> runs through n steps typing

each one, while <nT/> executes without typing. The option

V, with the same alternatives <nV> and <nV/> treats a

call or a conditional call as a single step, avoiding a

detailled examination of the interior contents of each

subroutine.

(V) A variant of T.

(X) Examine registers. Any one of the registers or

register pairs A, BC, HL, DE, SP, PC may be examined,

or one of the five flag bits C, Z, M, E, I, by typing

Xr, with r = czmeiabhdsp*, or =xxxx with a hexadecimal

address xxxx. The latter defines the register * at

first. A display line is typed, of the form

r=xxxx  xx ...  xx  ... xx  +++++  + +++++  ooo ooooo

where xxxx is the contents of register r, xx etc is

a window of hexadecimal bytes centered on xxxx, ++ etc

is a similar window of ASCII characters, and finally

ooo ooooo is the instruction, if meaningful, at xxxx.

Once an examination is underway, the window may be

moved by various operators:

*      indirect from window

^      indirect from instruction address

!      save value

?      recover value

/      next instruction

+      1 byte forward

-      1 byte backward

#      forward, stop at nonzero

@      backward, stop at nonzero

=xx    search for byte xx ('x or ^x also)

|xxxx  search for pair xxxx

>      forward half-window

<      backward half-window

.      type crlf - make new line

################################################