BEACONS V1.1

by John Augustine N3AOF

"Beacons" (freeware) was written in assembly for Dos platforms.  It can also

run via Windows at the Dos prompt or via "Start" menu, "Run".  Windows XP,

launch "COMMAND" via "Run" to access Dos commands.  "Beacons" uses less than

3K bytes and no "installation" is required.

"BEACONS" PURPOSE:

This program can be very useful to "ham" radio operators as well as swl

(short wave listeners).  The purpose of "Beacons" is to simplify the

identification of radio beacons around the world.  Otherwise, you would

require a skill in Morse code.  Even if you are an expert at Morse code, this

program has the advantage, because it can show you the source of a beacon/s

when the signal is barely perceivable.  Not only that, it shows you the beacon

source/s you can't hear and the location/s that are not "open" to

communications or reception.

RADIO BEACONS:

Thanks to Larry, W8VLN, for providing the following beacon information used

in "Beacons":

The beacons transmit every three minutes, day and night. This table gives

the minute and second of the start of the first transmission within the hour

for each beacon on each frequency. A transmission consists of the callsign of

the beacon sent at 22 words per minute followed by four one-second dashes. The

callsign and the first dash are sent at 100 watts. The remaining dashes are

sent at 10 watts, 1 watt and 100 milliwatts.

-------------------------------------------------------------

Call   14.10 18.11 21.15 24.93 28.20 Location - Operator, Status

- - - | - - | - - | - - | - - | - - | - - - - - - - - - - - -

4U1UN  00:00 00:10 00:20 00:30 00:40 United Nations - UNRC OK

VE8AT  00:10 00:20 00:30 00:40 00:50 Canada - RAC/NARC OK

W6WX   00:20 00:30 00:40 00:50 01:00 U.S. - NCDXF OK

KH6WO  00:30  OFF  00:50  OFF  01:10 Hawaii - NOARG/HARC OK

ZL6B   00:40 00:50 01:00 01:10 01:20 New Zealand - NZART OK

VK6RBP 00:50 01:00 01:10 01:20 01:30 Australia - WIA OK

JA2IGY 01:00 01:10 01:20 01:30 01:40 Japan - JARL OK

RR9O   01:10 01:20 01:30 01:40 01:50 Russia - SRR Garbled

VR2B   01:20 01:30 01:40 01:50 02:00 Hong Kong - CRSA/HARTS OK

4S7B   01:30 01:40 01:50 02:00 02:10 Sri Lanka - RSSL Intermit

ZS6DN  01:40 01:50 02:00 02:10 02:20 South Africa - ZS6DN OK

5Z4B   01:50 02:00 02:10 02:20 02:30 Kenya - ARSK OK New location

4X6TU  02:00 02:10 02:20 02:30 02:40 Israel - IARC OK

OH2B   02:10 02:20 02:30 02:40 02:50 Finland - SRAL OK

CS3B   02:20 02:30 02:40 02:50 00:00 Madeira - ARRM OK

LU4AA  02:30 02:40 02:50 00:00 00:10 Argentina - RCA OK

OA4B   02:40 02:50 00:00 00:10 00:20 Peru - RCP OK

YV5B   02:50 00:00 00:10 00:20 00:30 Venezuela - RCV OK

GETTING STARTED:

It is very important to set the computer's time accurately.  Using Dos

command, type "TIME" and press Enter.  You have the option to enter a new

time.  Enter a time (example for 5:10PM) type (without quotes) "17:10" a

minute before the actual time of 5:10PM arrives while listening to WWV on your

radio, or using a precision time reference.  As soon as you hear the tone,

press the Enter key.  This will set the computer's clock to exactly 17:10:00

when you press Enter.  Afterwards, you can launch "Beacons" at any time.

MISC.

Computer's clocks often don't keep precision time after a day or more

passes.  I recommend that you check the time if you use "Beacon" a day or more

later, to verify that the time is not more than a second or two from the exact

time.  If it is, reset the time again.

If a computer (laptop) goes to "sleep" while "Beacons" is running, and you

"wake it up", it is very possible that the program will be "out of step"

(error).  Simply press any key to abort the "run" and select the band you are

monitoring to get back in "step".

If you have any problems with "Beacons", please let me know.  Compliments

are also welcomed.

John Augustine N3AOF

jaugust@bellatlantic.net