Compiling New PCbridge code

The PCbridge distribution comes with precompiled versions of the code

for the most common bridge configurations.  If your configuration is not

one of these common ones, or if you want to modify the code in some

special way, you will need to recompile it.  This document describes how

to do this.

WHAT YOU WILL NEED.

The original source code was written using TURBO ASSEMBLER 1.0.  I have

made no attempt to check if the code will assemble with other assemblers,

but you are welcome to try.  Here I will assume that you have the TASM and

TLINK utilities that come with TURBO ASSEMBLER.   In addition, the

compilation needs a FULL 640K of memory in order to compile.  Thus you

need a PC with at least this much memory and you will probably have to

remove any TSR programs so that TURBO ASSEMBLER has as much space as possible.

CONFIGURING PCbridge.

In order to change the interface configuration of PCbridge it is only

necessary to change one file, BRIDGE.ASM.  Near the top of this file

declaration section that contains a description of each card in the bridge

and it is this description that needs to be changed.

------------------------------------------------------------------------------

WD Ethernet Cards:

The easiest way to describe what is involved with configuring a WD ethernet

interface is simply to look at an example.  Here is the declaration for

the first WD8013EBT card.

WDE_DECLARE %cur_if, 280H, 0D000H, 0H, 1, 64, 1  ;; promiscuous ,16K, 16bit

IF_DECLARE %cur_if, %cur_if, WDE

cur_if = cur_if + 1

The first line declares a WD800X card.  The meaning of of the

parameters to the WDE_DECLARE statement is

Parameter

1       The name of the card.  This never changes, it is always %cur_if

2       Is the I/O address of the card.  This number is basically

arbitrary as long as it does not conflict with anything and

you set the DIP switches on the WD800X card to agree with

this number.  In general I choose 280H for the first card

2A0H for the second card, 2C0H for the third card etc.

I recommend this pattern.

3,4     This is the address (segment, offset) of the shared memory.

It must be above 80000H (512K) and it can't conflict with

other device memory (like video memory).   Most PCs have

a free region D0000H-DFFFFH and A0000H-AFFFFH, so I recommend

using these.  To simplify configuration I give each WD800X

card a region that has a full 32K of memory even though the

WD80013EBT and the WD8003E will not use all of it.  Thus I

reserve D0000H-D7FFFH for the first card, D8000H-DFFFFH for

the second A0000H-A7FFFFH for the third etc.

5       This flag, that indicates promiscuous mode, is always 1

for bridges.

6       This is the number of 256byte 'pages' of shared memory

on the card.  WD8003E have 8K = 32 pages, WD8003EBT have

32K = 128 pages, and the WD8013EBT have 16K = 64 pages.

7       This flag, that indicates a 16 bit data path, is 1 for

the WD80013EBT card.  Otherwise it is either 0 or blank.

The second line declares that the code for the WD800X implements a IF

software interface and that the names of the functions all begin with the

prefix 'WDE'.   This line never changes.

Thus to configure Ethernet (or Starlan) cards, all you need do do is

have a set of three lines like the ones above with the proper parameters

to the WDE_DECLARE line.  There are templates in the BRIDGE.ASM file that

have been commented out for various common cases.  Simply modify the

template that is closest and comment it back in.

------------------------------------------------------------------------------

3Com 3c507 Ethernet Card:

The declaration for the 3c507 card looks like

C507_DECLARE %cur_if, 280H, 0D000H, 10000H, 1   ;; name,io,seg,len,prom

IF_DECLARE %cur_if, %cur_if, C507

cur_if = cur_if + 1

The first line declares a 3C507 card.  The meaning of of the parameters to

the this statement is

Parameter

1       The name of the card.  This never changes, it is always %cur_if

2       Is the I/O address of the card.  This number is basically

arbitrary as long as it does not conflict with anything and

you configure the 33c507 card to to agree with this number.

In general I choose 280H for the first card 2A0H for the

second card, 2C0H for the third card etc.   I recommend this

pattern.

3       This is the SEGMENT address of the shared memory.   (Thus

the value D000H cooresponds to D0000H in the address space).

Most PCs have a free region D0000H-DFFFFH and C0000H-CFFFFH,

so I recommend using these.   Note you should probably

check with using 'debug' that these regions are in fact

clear if you have any doubt.  This value must agree with

the value set in the card.

4       The Length of the shared memory.  The 3c507 has 64K of ram,

but can be configured to use less.  This is only necessary

if you are using more than two 3c507 cards.  In this case

there is not enough address space for all the memory on all

the cards.  Thus you should probably change this number

to 32K (8000H), when this is the case.  This value must

agree with the value set in the card

5       This flag, that indicates promiscuous mode, is always 1

for bridges.

The second line declares that the code for the 3C507 card implements a IF

software interface and that the names of the functions all begin with the

prefix 'C507'.   This line never changes.

------------------------------------------------------------------------------

Serial Cards:

The declaration for a serial interface is more complicated.  It looks

like this.

;; a serial card for a remote bridge

I8250_DECLARE %cur_if, SLIP, %cur_if, IBM_COM1_PORT, IBM_COM1_IRQ, 0

tmp = cur_if*100                                 ;; generate temp names

BUFF_DECLARE %(tmp+1), 3200                      ;; the read buffer

QUEUE_DECLARE %(tmp+2), 5, %(size qif_entry)     ;; the read queue

BUFF_DECLARE %(tmp+3), 10240                     ;; the write buffer

QUEUE_DECLARE %(tmp+4), 16, %(size qif_entry) ;; the write queue

SLIP_DECLARE %cur_if,%cur_if,%(tmp+1),%(tmp+2),%(tmp+3),%(tmp+4)

MY_SET_MTU %cur_if, 1518                         ;; overide default MTU

Q_IF_DECLARE %cur_if,%(tmp+1),%(tmp+2),%(tmp+3),%(tmp+4),SLIP,%cur_if

IF_DECLARE %cur_if, %cur_if, Q

cur_if = cur_if + 1

Luckily, the ONLY line you will ever need to modify is the I8250_DECLARE

line and even on that line you will ever want to change parameters 4-6.

Lets look at these parameters more closely.

Parameter

4       This is the I/O address of the serial port.  There is

an standard place for the first 4 serial ports on the

PC, and these have been given symbolic names of the

form IBM_COM<number>_PORT.  Use these names if you can,

or a hexidecimal number if the port is in a non-standard

place.

5       This is the Interrupt line (IRQ) for the serial port.

There are standard interrupt lines for the first 2 serial

ports and these have been given names IBM_COM_<number>_IRQ.

Note that standard IRQ for COM1 is 4 and the standard

IRQ for COM2 is 3.  Often other non-standard hardware will

use these interrupt lines, so be sure that they are free

before you use them.

interrupt line

6       This flag indicates if hardware flow control should be

used.  If this flag is 1, then PCbridge will only send

out characters if the RS232 signal CTS (clear to send)

is asserted.  PCbridge does not need flow control because

it can accept characters at the full baud rate, but

it is possible that you are connecting the PCs with modems

that can't handle the full baud rate all the time (compressing

modems for example).  By enabling this flag, and setting

the modem properly, this feature will make sure PCbridge

sends characters only as fast as the modem can transmit them.

------------------------------------------------------------------------------

The Packet driver

It is possible to use PCbridge with ethernet cards other than the

Western Digital ones by using the packet driver.  Unfortunately, this

relatively slow and since EVERY packet has to be examined, it is very

likely that many packet will be dropped.  Nevertheless some people may

want to experiment with it, so here is the declaration.

PKT_DECLARE %cur_if, 60H, 01, 1    ;; <name> <Int num> <Class (Eth=1), prom

IF_DECLARE %cur_if, %cur_if, PKT

cur_if = cur_if + 1

The only parmeter that might change is the second one to PKT_DECLARE.

This parameter (in this case 60H) is the software interrupt used to

access the packet driver.  For EACH packet driver interface there must

be a packet driver loaded that will respond to this interrupt.

=============================================================================

Compiling the software:

Once the BRIDGE.ASM file has been configured, compiling it could not

be easier.  Simply type

TASM bridge

TLINK bridge

and a new version of the bridge code called bridge.exe will be produced.

=============================================================================

Debugging:

For those of you that are modifying the code and wish to print out

debugging statements this is possible.  The file DEBUG.INC contains

routines for doing just that.  The main routines are PRINT, PRINT_REG

and DUMP_in_SI_CX.  PRINT simply prints a string.  PRINT_REG prints

a string and the contents of any 16 bit register (in hex).  DUMP_in_SI_CX

prints CX memory locations starting at SI using the named segment

register.  All of these routines modify NO registers so they can be

used anywhere (except between a cmp and its jump instr).

Here is an example of how these macro look when they are called

print <Starting program>

print_reg <SI = >, SI

print_reg <CX = >, CX

print <Dumping at SI for CX bytes>

dump_in_SI_CX DS