PCbridge source code description

Vance Morrison

PCbridge is distributed with source for essentially one reason, I would

like to involve many people in the design and debugging of PCbridge, and for

them to do this, they need the source.   Since I will not be the only one

fixing bugs and adding enhancements, PCbridge will continue to develop as long

as there are people who think it is a worthwhile program to support.  As

creator of PCbridge, however I request/demand the following

1) Please let me know about any bug fixes so that I can incorporate

them into the 'official' release.

2) You may make local enhancements to PCbridge and use them LOCALLY

(that is within the same organization) but you may NOT distribute

this code.

The reason for the second demand (it is in the copywrite) is very simple,

I don't want PCbridge to fragment into many versions.   That is my ONLY

concern.  Thus just tell me what you want to do, and I will give you

an copy of the working source code, and you can be sure that the part that

you are working on is not being worked on by anyone else, and when you are

done it will be ready to integrate into the official release.   I do

reserve the right NOT to integrate features that I regard as counterproductive

and code that simply is too poorly written or buggy to risk, but I do not

see myself excercising that option.

All of these conditions are simply to insure that PCbridge has a long

and productive life.   I have no monetary interest in PCbridge.

============================================================================

DESIGN CRITERIA

PCbridge had to be designed to squeeze all the power it could out

of an 8088 CPU.  As such I had to make some rather drastic measures.

These include

1) Programming the main packet routing loop in assembler

2) NO procedure call in the main packet routing loop

Since assembler have very good macro expansion capabilities, the

second decision is relatively painless.  Simply use macros instead

of procedure calls.   This can make the code large, but memory has

not been a problem yet.

Once we are out of the main packet loop, these optimizations are

not really necessary anymore.  Thus if/when SNMP support is added

to PCbridge, it will be done in C, since all of this code is only

executed when SNMP operations are being done.

===========================================================================

CONVENTIONS

Despite the use use of assembler, I have tried to make the code

as modular as possible, and to adopt conventions that I have found quite

useful in preventing common assembly code errors.   Some of these

conventions are

1) function prefixing - Macros are groups into modules that provide

a set of related services.  All function in this group are

prefixed by the same short string.  This improves readability

and prevents name conflicts

2) Naming input, output and constant registers - The most common

error in assembly code is calling a macro that changes registers

that you believed where held constant.  To prevent this type

of error Macro names have the following form

<NAME>_in_<REG LIST>_out_<REG_LIST>_const_<REG_LIST>

For example

WD_IF_COPY_in_CX_SI_DI_ES_out_SI_DI_const_BX_BP_ES name

It is quite clear from the name that this macro has its parameters

passed in CX,SI,DI,ES, it returns values in SI,DI and it holds

the registers BX,BP,ES constant.  While this may seem long-winded

(it is) it is VERY useful and has prevented many bugs and I

will DEMAND that any new assembly code follow these conventions.

Note that since the CS,DS,SS registers are NEVER changed, you

may assume that EVERY macro preserves these registers.

3) file naming - There are two type of files in the source code at

present, INC files and ASM files.  INC files contain ONLY macro

definitions and admit NO code and are meant to be included in

ASM files.  INC file are thus the 'meat' of the code.  The ASM

files contain NO macro definitions, but do instantiate the macros

to generate the actual code.   At present there is only one

ASM file (BRIDGE.ASM).

===========================================================================

The IF interface:

The IF interface is a device-independant view of a network card.  Thus

new hardware can be supported simply by writing code to implement an

IF interface for the new hardware.

IF.INC          Supplies a standard interface for all card specific

code and dispatches the proper code when generic

routines are called.

===========================================================================

IF interfaces for 3COM 3c507 Ethernet:

The following file implement a IF interface for 3COM 3c507 card

3C507.INC       IF interface specific to the 3c507.INC file

===========================================================================

IF interfaces for WD Ethernet and Starlan code:

The following files implement a IF interface for Western digital cards

WD8003E.INC     IF interface specific to the WD8003E, WD8003EBT

and WD8013EBT cards.  It also works with WD8003S

and WD8003SH starlan cards, but the buffering

is not optimized for these slower cards.

WD.INC          Constant declarations needed by the above two

modules to manipulate the WD8003 card.

==========================================================================

IF interface for the packet driver

PACKET.INC      Implements an IF intererface for FTP software's

packet driver specification.   Since many packet

drivers have been written for may cards, this

module immediately provides support for all these

cards.  Unforunately, the packet driver specification

is not highly optimized for throughput, thus this

interface is usually 1/3 the speed of a 'native'

IF interface.

==========================================================================

IF interface for a serial COM port

I8250.INC       Implements code for getting and putting single

characters on and off a 8250 serial chip.  It

also has support for the 166550AF chip with FIFOs.

This module is the only one that has to deal with

the vugarities of setting interupts and dealing

directly with the serial chip.

SLIP.INC        The stream of input characters from the I8250

interface and packatizes them according to the

SLIP protocol.  Also adds packet markers to

outgoing packets producing a outgoing character

stream that the I8250 module transmits.

QIF.INC         Impelents the 'other half' of the packet queuing.

It is this code that queues up incomming and

outgoing packets.  It looks like a IF interface

to higher levels.

==========================================================================

The Bridge Code:

BRIDGE.INC       This file implements the actually packet bridging

algorithm.  In addition, it also acts like a IF

interface.  The idea is that any packets that are

not addressed to the bridge are inspected and

bridged if necessary, however if a packet is sent

directly to the address of the bridge this packet

is sent to the higher level (via an IF interface)

for processing.  This allows the possibility for

Bridge-routing or network control (SNMP).

BRIDGE.ASM        The 'main program' for the bridge.  This file

is only one you should need to change to reconfigure

the bridge (see compile.doc).

===========================================================================

Support routines

In addition to the major functional blocks, there are some modules that

provide rather generic services that are used by the other functional blocks.

These modules include

DEBUG.INC       Provides simple routines that print to screen routines.

They should not be in production code (use log routines)

These routines violate convention in that they

preserve ALL registers but do not have a CONST

part in their name.

BUFFER.INC      Provides simple buffer allocation routines used in

output (and possibly input) queuing.  Used in

QIF.INC ATALK.INC and WD8003.INC

QUEUE.INC       Provides simple queuing routines used for packet

queues.  Used in ATALK.INC and WD8003.INC

==========================================================================