SPANStorÔ Quick Start

SPANStor software is easy to install and administer.  Just follow the steps listed below to get your SPANStor storage server up and running quickly.

Before you begin:

SPANStor requires a PCI based, X86 architecture computer with a floppy drive.  In this release, an Intel Pro100B 10/100 Base-TX PCI Ethernet adapter and an Adaptec 7800 family PCI SCSI controller are required.  SPANStor will provide the best performance with an AHA-2940 Ultra SCSI controller, and at least 64MB of RAM memory.   Network Storage Solutions recommends 256 MB of main RAM memory for optimal performance.  A Pentium based CPU is typically sufficient to provide high performance.

WARNING: When SPANStor loads and you choose to ZAP (initialize) a disk, any data contained on the disk will be lost.  Insure that no SCSI disks in units used for SPANStor contain data that cannot be deleted.  The trialware version of SPANStor only supports 1GB of storage.  The disk drive with the lowest SCSI ID will be initialized.  For data safety, disconnect  any other SCSI disks in the computer other than the disk to be utilized.  If a disk larger than 1GB is used, only 1GB will be recognized.  The registered version has been tested with up to 126GB of storage capacity.

Quick Install:

- If you downloaded the Trialware version of SPANStor, you will need to create a SPANStor boot floppy. A formatted, 1.44MB floppy is required. Begin by downloading the installation file to a temporary directory.  For DOS/Windows, run the self-extracting executable SPANSDOS.EXE to automatically extract the required files, then run INSTALL.COM in a DOS command window. If you downloaded the UNIX tar formatted file spanstor.tar,  use the following commands to build the boot floppy:

- #tar -xvf spanstor.tar

- #dd of=/dev/rfd0 if=spanstor.dd obs=512      (your raw floppy device may vary)

- If you wish to setup the server without using a console or serial terminal, the config.586 file on the floppy should be edited to include the desired IP address, Netmask, and other settings. See the command line switch settings example at the end of this document.

- If the SPANStor computer does not have a keyboard and VGA monitor, connect a serial terminal to the COM1 port on the SPANStor computer.  Communication settings 9600,N,8,1.

- Insert the boot floppy in the computer you wish to use as a high performance storage server and boot the unit.

- As the unit is booting you will see various initialization messages on the VGA console or serial terminal. When the screen displays the first countdown  “Type any key to enter the Hardware Menu 5.4.3…”  press any key.   The following screen will be displayed:

+———————————————————————————————+

|  Hardware Initialization Menu |

+———————————————————————————————+

*** WARNING  WARNING  WARNING ***

*** Use of items in this menu ***

***   may destroy your data   ***

<R> Reset

<S> Show SCSI

<Z> Zap Disk

<N> Continue with Normal Startup Menu

<Q> Quit

- Select <Z> to initialize the SCSI disk.  When prompted to initialize the whole disk, you may answer “n” to perform a quick initialization.

- After the initialization completes, select <N> to proceed with a normal startup.  When the second countdown message is displayed,  “Type any key to enter menu .10.9.8.7... “

Press a key to interrupt the countdown and display the Configuration Menu.

The Configuration Menu now displays:

+——————————————————————+

|  Configuration Menu  |

+——————————————————————+

<N> Network Configuration

<A> Server Administration

<C> Show Configuration

<B> Backup Configuration

<S> Show SCSI

<P> Change System Performance Parameters

<V> Volume Information

<W> Statistics

<U> Show Server Up Time

<Q> Quit

- SPANStor software requires a license key.  To obtain a license key fill out the attached FORM and FAX it to (703) 834-0787,  alternately send EMAIL to: license@nssolutions.com providing the same information that is on the form, and a key will be sent to you within 4 business hours.  You will need the Ethernet MAC address displayed on the SPANStor screen (six double digits separated by colons), or you can display the address by choosing <C> Show Configuration from the Configuration Menu.  Once you have received your software key, choose <A> Server Administration and enter the key.

- At this point, you may configure any other parameters you desire to change from the defaults, or simply <Q> to quit and the SPANStor storage server will come up.

- To re-enter the menus after the system has started you must press the attention key, <?> from the console, and enter the system password.  The default password is “password”, which can be changed from the <A> Server Administration Menu.

If  an IP address and Netmask are set from the config.586 file on the floppy, or DHCP is enabled to automatically assign an IP address and netmask, the SPANStor configuration menus can also be accessed by pointing  a Web Browser, or Telnet connection to the IP address.  You will still be required to enter the system password.

Common CONFIG.586 command line switches (see Appendix D in the manual for a complete listing)

Example: 	# SPANStor 586 program - set startup options via cmd line

title= SPANStor

kernel= /spans586.out -ip 198.98.0.15 -netmask 255.255.255.0 -fast -smbname NT01

#pause= Load Completed - press any key to continue

-ip		IP address in dot notation form

-netmask	subnet mask in dot notation form

-fast	starts menu countdown at 2 seconds vs. 10 sec.

-smb	used to set the SMB/CIFS (Windows)  servername

-nosmb	disables the SMB/CIFS protocol

-syncnfs	use synchronous (immediate) NFS writes

-asyncnfs	use asynchronous (delayed) NFS writes

-nonetbios	disable browser view of servername (hide server)

-netbios	enable Windows browser view of server

-c		set initial console = keyboard

-s		set initial console= serial port

-startup	unconditional startup (Requires either -c or -s)

-hwinit	hardware init

-export	enable use of exports file

-noexport	disable use of exports file

-bootp	enable bootp protocol

-nobootp	disable bootp protocol

-rarp	enable RARP (Reverse Address Resolution)

-norarp	disable RARP

-backup	enable local backups

-nobackup 	disable local backups

-http	enable HTTP (Hyper Text Transmission Protocol)

-nohttp	disable HTTP

-license 	license string characters

| FAX | KEY REQUEST |
| --- | --- |

| Date: |  |  | You may request a license key by EMAIL |
| --- | --- | --- | --- |
| # of pages including cover sheet |  | 1 | Send your request containing the required fields to: license@nssolutions.com |

| Name: | NSS License Administrator |  |
| --- | --- | --- |
| Company Name: | Network Storage Solutions Inc. |  |
| Subject: | SPANStor Key Request |  |
| Phone: | 703-834-1800 |  |
| Fax: | 703-834-0787 |  |
| EMAIL: | license@nssolutions.com |  |

| Name: * |  |
| --- | --- |
| Company:* |  |
| Address 1:* |  |
| Address 2: |  |
| City,State* |  |
| ZIP Code:* |  |
| EMAIL: |  |
| Phone:* |  |
| Fax:* |  |
| How did you find about SPANStor?* |  |

Items marked with an * are required fields.  Missing information will delay processing.

Visit our web site:  http://www.nssolutions.com

for the latest updates to SPANStor software and documentation.

Technical support for Trialware software is only available by sending EMAIL to: support@nssolutions.com

Purchase a registered license now for installation support, increased storage capacity support, increased HTTP connections, Tape backup support, and other advanced features.

| Ethernet Address: * | : : : : : |
| --- | --- |
| Computer Make |  |
| Storage Capacity |  |
| Network Protocols Used (circle) | IPX/SPX NFS SMB/CIFS HTTP DHCP RARP BOOTP 100 Mb 10 Mb |
|  | ¯ To be filled out by Network Storage |
| KEY = |  |

Comments:

|  | Mark this box to have a salesman call |
| --- | --- |