_ __   __   ()       _ __    _    __   _    ,

' )  ) /  `  /\      ' )  ) _//   /  ) ' )  /   Sound Sample Utility

/--' /--   /  )      /--'  /    /--/   /  /    Software for the IBM

/  \ (___, /__/__    /     /___ /  (_  (__/_    PCs useless speaker!

//

Version 1.00     (/       (C) Mark J Cox  1990

_____________________________________________________________________________

RESPLAY is a memory resident program designed to help high-level language

programmers make the most of the PCs useless speaker.  By use of some

simple procedures, you can playback samples from within your programs

and sample yourself.  Playback can either be to the PC speaker (no too

bad) or to some external hardware (described later). This program is

designed as a sister to the PLAY (currently PLAY400) series - this is

a sound sampler program for VGA with lots of nice features - If you

have not yet got a copy, try getting it from the BBS mentioned near the

end of this documentation.

_____________________________________________________________________________

POSSIBLE USES OF RESPLAY

------------------------

o Arcade Games ("He's Dead Jim!" or realistic shooting noises)

o Sound Analysis (Fast Fourier Transforms/ analysing spoken words)

o Spelling Games for kids.

o Sampling your friends and making them sound silly (by changing

their speed, or by FFTs and altering their pitch!)

o The next 'Jive Bunny' single

Features

========

o Simple to call from any high or low level language

o Options for playing through Speaker or external hardware

o Total control over speed and length of samples.

o Samples can be as big as there is free memory

o Freeware (more on this later)

_____________________________________________________________________________

PUBLIC DOMAIN - FREEWARE

------------------------

Please enjoy the RESPLAY program, you can give it to anyone

and use it for anything that does not include commercial gain.

(IE you can give it away with any Freeware program you create,

but not with SHAREWARE ones - where you are asking a fee for

your program, or commercial ones)

If you want to use RESPLAY in a Shareware or Commercial package

please contact the author who will be happy to negotiate a license!

The only stipulation is that the program remains unchanged and

that it is credited in your documentation (you need not include

this documentation with your program)

Why did I make it FREEWARE (I've had a couple of offers from

software companies for this!)? Well, because I'm a student and

hate software that I have to pay for, then find it is useless or

I could have written better... Shareware is a bit better, letting

you evaluate the package - but it's still a costly business.. By

making this software totally free I hope more people will enjoy

it and use it in their programs.

______________________________________________________________________________

INSTRUCTIONS FOR USE

====================

Resplay is a memory resident program.  That means that when you first

run Resplay it stores a bit of itself (only 1.5k) in memory that can

then be called by other programs run later.  To use the Resplay routines

you simply have to load the register set with the values show below and

call the interrupt 2F.  If you know what I am talking about, then you

should be able to program using Resplay immediately - otherwise have

a look at the example file, EXAMPLE.C.   Your programming manual

for your favourite language should explain all about how to call Interrupts

and set registers - you can do it in all the Borland TURBO packages and

just about everything else I can think of.  When I get time I will

produce example programs in this package for C, BASIC and PASCAL from

a variety of manufacturers.

The first thing to do is to load RESPLAY

C:\AMAZING\PROGRAMS>  RESPLAY

There are four main routines that you will need to call, these are

described below:

1. Check if Installed:

Set AX to 08201h (Installation Check)

Call Interrupt 2Fh

If AX contains 7746h then RESPLAY is installed.

otherwise RESPLAY is not installed.

2. DeInstall:

Set AX to 08202h (De Installation)

Call Interrupt 2Fh

IF AX contains 1000h (Sucess) then RESPLAY was deinstalled

otherwise RESPLAY was not already installed.

To sample or to play, the first thing to be done is to send

some setup parameters to the SET UP routing

3. Setup:

Set AX to 8210h (Set Up Routine)

Set CL to 0 for PLAY mode, 1 for SAMPLE mode.

Set BL for what port/speaker to use

BL = 0 ;  Use Printer Port (LPT1)

BL = 1 ;  Use Printer Port (LPT2)

BL = 2 ;  Use Prototype board at 0300h

BL = 3 ;  Use Printer Port (Alternative LPT1)

BL = 4 ;  Use internal SPEAKER

Set BH for the sample/playback speed ( divided by 250 )

(BH = 44 for 11kHz, 45 for 11.25kHz etc)

Call Interrupt 2Fh

IF AX contains 1000h (Sucess) then all was ok and has been

set up

IF AX contains 2000h (Failure) then a parameter was out

of range (CL=0,1;BL=0..4;BH=20...160)

otherwise RESPLAY is not loaded.

Then when you actually want to do the sample/playing:

4. Main:

Set AX to 8200h (Main Bit)

Set DX to the Start Segment of the sample space

Set DI to the Start Offset of the sample space

Set CX to the High word of the length

Set BX to the Low word of the length

Call Interrupt 2Fh

If AX contains 1000h (Sucess) all went ok

IF AX contains 2000h (Failure) then you have not called

setup first

otherwise RESPLAY is not loaded.

(Note length is not in Segment:Offset form it is simply a

32 bit word. 0003 3456 would be 33456 hex bytes)

Example:

--------

To Play back through the speaker at 11kHz the section of memory

starting at 4EF0:0100 for 135000 bytes (nearly 135k) then:

(Check to see if installed)

AX <- 8210 hex

BL <- 4         (speaker)

BH <- 44        (4 * 11kHz)

CL <- 0         (play)

Call Int 2F

(Make sure that it returns Sucess)

AX <- 8200 hex

DX <- 4EF0  \__start from 4ef0:0100

DI <- 0100  /

CX <- 0002  \__ 135000 bytes = 00020F58 in hex

BX <- 0F58  /              cx__////\\\\__bx

Call Int 2F

_____________________________________________________________________________

WHATS THIS HARDWARE THEN?

-------------------------

By addition of a piece of hardware costing around 20 quid (to build) onto a

spare printer port you can sample your own sounds and replay them through

your hi-fi with really astonishing quality.

HOW CAN I GET A CIRCUIT DIAGRAM?

--------------------------------

For a modest sum, I will send two 3.5" (Branded) High Density disks with

the latest version of PLAY and RESPLAY together with nicely printed

instructions and a copy of the circuit diagram.  To obtain this

package, send 10 pounds (sterling!) to the address at the end

of this document!  I make a little profit on each, but that will help

to cover the cost of the book I had to buy to improve my VGA routines!

HOW LONG A SAMPLE CAN I STORE?

------------------------------

It depends on what frequency the sample is and how much memory is free -

at 16kHz (the speed I like to sample at) I get about 33 seconds into

memory with no TSRs except a Disk Cache - I have a mac sample at 11kHz

of the entire Dr. Who. theme tune;  Sorry, until I fathom how

to use Extended/Expanded memory it can't use it!

GREAT BBS

---------

Try 'Kingdom of GREYHAWK' on Derby (0332) 756414 24hr for the latest

versions, PLAY350 the VGA interactive sampler and lots and lots of

samples - Don't forget to mention RESPLAY when registering!

(300...9600 baud)

MAIL ME?

--------

Please mail me about any bugs or future enchancements you

would like to see.  Leave a message on Greyhawk BBS

(2:250/404.5 if you use NetMail)

or "mjhc8@uk.ac.bradford.eeng" from any University/Poly etc.

(Normal Post does OK too! :-)

WARRANTY

--------

There is no warranty of any kind, and the copyright owner is not

liable for damages of any kind.  By using this free software,

you agree to this.

The software and documentation are:

Copyright (C) 1990

Mark J Cox

29 Lundie Close

Stenson Fields

Derby

DE2 3AN