This is a modification of the PC Magazine Utility "SETUP" and is being

distributed as required with the original files intact.  Take the time

to read them, or, if you simply want to have a fine Epson Resident

Printer Controller, use them.

If, however, you'd like to take advantage of the DMP2100P's unique

capabilities, use my changes.  You may also want to modify my source

code changes for your own Tandy printer, although most of the commands

will work across the Tandy line.  They are all designed to work in Tandy

mode, of course.

Here's what to expect:

I'm assuming your printer "powers up" to these defaults:

10 cpi

Data Processing Mode

Tandy control codes and character set

No auto line feed

These defaults are accomplished by setting all dip switches in bank 1

to off, except 1-6 (this is Tandy's recommendation when using the printer

with any MS-DOS machine, since DOS expects both a CR and LF at the end

of each line).

You can bring up the menu by hitting <CTRL>-<RSHFT> (Control and Right Shift

simultaneously).  It's really quite self-explanatory.  The first 8 choices

are toggles:  use the function key alone to toggle them on, use the function

key with a shift key (either one) to toggle them off.  Turning off any

special print modes with <SHFT> and <F1> through <F4> will reset the printer

to 10 cpi, draft mode (the default if your dip switches are set up as

described above).  Some of these sequences are for the beautiful letter

quality fonts resident in the 2100P.

<F5> sets the printer to Word Processing mode, in which it acts upon sub-

and superscript sequences immediately.  Read the manual on this; it's rather

confusing.  If your printouts don't look right in Data Processing mode (the

default), give WP mode a try.

<F6> selects 8 lines per inch, which looks best with 17 cpi but can crowd

extra lines in for doing labels or directory listings.

<F7> sets the auto linefeed mode to on, which is what former TRSDOS users

are accustomed to.  You'll get double spaced printouts if you turn this on

with MS-DOS packages.  It's there because I discovered that SCRIPSIT users

(either the 1000/2000 or PC versions) often buy the downloadable soft fonts

for the 2100P; I did.  If you're getting fancy and want to printout the

character widths and technical info that FEDIT supplies, it asks you if

you're in Tandy mode.  If you say yes, the program expects the printer to

auto linefeed!  I've been customizing my WordPerfect drivers to take advantage

of these downloadable fonts (see separate upload), and I needed to print out

the character tables.  This switch is much handier than turning off the

printer, throwing a dip switch, and then turning it back on.  It's also much

nicer to the power supply!

<F8> toggles between the Tandy character set and the IBM set.  Although you

can use either character set in both the Tandy and IBM emulation modes, this

escape code sequence only functions in Tandy mode.

<F9> sends a form feed (ASCII 12), which works in both Tandy and IBM mode.

<F10> toggles the 2100P's mode between Tandy codes and IBM codes, which I find

handy for those software packages that don't properly support this printer.

Only <F9> is valid in IBM emulation mode; the other sequences will either

print the hourglass (invalid code) figure or give even more unpredictable

results.

I believe that all the codes, except those relating to the high quality fonts,

will work with the DMP130, DMP130A, DMP430, DMP2110, and DMP2200.  Of course,

some of these printers have other features, such as italics, that my program

doesn't support.

Feel free to make any modifications to my source code, but please adhere to

PC Magazine's copyright restrictions and distribute the original program

intact with its documentation.  I'm sure they wouldn't want you to charge

anything, either!  While we're on that subject, do NOT offer me any monetary

contributions for my fiddling.  DO offer constructive suggestions and report

any problems.  I've been using the program with other TSR's for several

months and haven't had any problems.  HEED THE CAUTIONS GIVEN BY THE ORIGINAL

AUTHOR.

Finally, just to keep everything legal, I am offering this program to you

free of charge, but I do not claim it does anything.  I am also not

responsible for any damage it may cause, monetary or otherwise, to your

equipment or your data.  Use it at your own risk; doing so is evidence of

your acquiescence to these terms.

I can't promise any technical support, but you can certainly leave me

messages on CompuServe (72265,421), Delphi (JOHNBSIENTZ), or The Source

(BEM533), and I'll try to solve any problems you might have.